Oak Tree House, Alan Farnaby Way, Sheriff Hutton Industrial Park,

York Road, Sheriff Hutton, York, YO60 6PG

Tel: 01347 878972●E-Mail: info@whlgroup.net●www.whlbuildingservices.co.uk

CONTRACT NAME:			Portwest Clothing

CONTRACT No.			C2206

QUOTE REF No.			Q008

ESTIMATE No.			E21047

SITE ADDRESS:			Fields End Business Park, Thurnscoe S63 0JF

MAIN CONTRACTOR:		Simpson York Ltd

Contract Manager:			Mark Cregan

QS:					James Fuller

STEEL CONTRACTOR:		Gravica

Contact:				Daniel Cox

ARCHITECT:				WHP Architecture

Contact:				Tom Edwards

Contract Deed:		Yes		Period		12 Years

SITE TREATED AS: 			INLAND

THE BELOW SPECIFICATION IS TO BE READ IN CONJUNCTION WITH: Q008

ROOF TYPE: COMPOSITE

EXTERNAL SHEET PROFILE:			RW

EXTERNAL GAUGE:				0.5mm

EXTERNAL COLOUR:				Goosewing Grey

EXTERNAL COATING / GUARANTEE:	XL Forte

LINER PROFILE:				Lightly planked

LINER GAUGE:				0.4mm

LINER COATING:				Cleansafe 15

CORE THICKNESS:				100mm

CORE TYPE:					Quadcore

COMPOSITE MANUFACTURER:		Kingspan

ROOF U VALUE:				0.18 W/m²K

ROOFLIGHTS

ROOFLIGHT TYPE:   				FAIRS

ROOFLIGHT %:				10%

OUTER ROOFLIGHT:				3.0kg

INTERMEDIATE:				10mm Polycarbonate

INNER ROOFLIGHT:				1.5kg

ROOFLIGHT U VALUE:			1.2 W/m²K

ROOFLIGHT L VALUE:			36%

ROOFLIGHT G VALUE:			36%

ROOFLIGHTS FRAGILITY:			25 Years non fragility

MANUFACTURER:				Hambleside Danelaw

ROOF FIXINGS AND FLASHINGS

TO MATCH CLADDING (Y/N):		Yes

FLASHING COLOUR:				Goosewing Grey

FLASHING COATING / GUARANTEE:	XL Forte

FIXINGS / GARANTEE:			Stainless Steel / 25 Years

SEALANTS / GUARANTEE:			Enhanced / 25 Years

GUTTERS

GUTTER TYPE:				Trimline external

MATERIAL:					0.7mm pre-coat steel

COLOUR:					Goosewing Grey

COATING:					HPS200 Ultra

DRAINAGE SYSTEM

SIZE:						150mm x 150mm

MATERIAL:					0.7mm pre-coat steel

COLOUR:					Goosewing Grey

COATING:					HPS200 Ultra

QUANTITY:					To suit gutter calculations

LOCATION:					To suit gutter calculations

COMPOSITE WALL CONSTRUCTION (HIGH LEVEL)

EXTERNAL SHEET PROFILE:			Trisomet

ORIENTATION:				Vertical

MODULE WIDTH(S): 			1000mm

EXTERNAL GAUGE:				0.5mm

EXTERNAL COLOUR:				Goosewing Grey

EXTERNAL COATING/GUARANTEE:	HPS200 Ultra / 40 Years

LINER PROFILE:				Lightly planked

LINER GAUGE:				0.4mm

LINER COATING:				PE15

CORE THICKNESS:				80mm

COMPOSITE MANUFACTURER:		Tata

U VALUE:					0.25 W/m²K

COMPOSITE WALL CONSTRUCTION (‘PICTURE FRAME AREAS’)

EXTERNAL SHEET PROFILE:			Trisomet

ORIENTATION:				Vertical

MODULE WIDTH(S): 			1000mm

EXTERNAL GAUGE:				0.5mm

EXTERNAL COLOUR:				Albatross / Merlin Grey

EXTERNAL COATING/GUARANTEE:	HPS200 Ultra / 40 Years

LINER PROFILE:				Lightly planked

LINER GAUGE:				0.4mm

LINER COATING:				PE15

CORE THICKNESS:				90mm

COMPOSITE MANUFACTURER:		Tata

U VALUE:					0.25 W/m²K

WALL FIXINGS AND FLASHINGS

TO MATCH CLADDING (Y/N):		Yes

FLASHING COATING:			HPS200 Ultra

FLASHING COLOUR:				To match the adjacent cladding

FIXINGS / GUARANTEE:			Stainless Steel / 25 Years

SEALANTS / GUARANTEE:			Enhanced / 25 Years

OPENING INSULATION

GOODS DOORS:				75mm

PERSONNEL DOORS:			25mm

WINDOWS:					25mm

INSULATION TYPE:				PIR

TECHNICAL

FIREWALL GRID LINES:			GL J1-19 and J19 - E19

FIREWALL SPEC:				60 minutes

AIR TIGHTNESS TO BE ACHIEVED: 	5.0 m³/ (h.m²) at 50 Pa.

GUTTER CALCULATIONS: 			Based on 1 year return period

OTHER

ROOF PENETRATIONS:			N/A

SHELF ANGLE:				N/A

LOUVRE TYPE:				N/A

PLEASE SIGN AND DATE BELOW TO CONFIRM THE ABOVE SPECIFICATION IS SATISFACTORY:

SIGNED BY: _________________________________________________________________

COMPANY: _________________________________________________________________

DATE: _______________________