# RSA decision Log RESPONSE

| Revision | Date | Author | Checked by; | Comments |
| --- | --- | --- | --- | --- |
|  | 04/08/26 | Leigh Ogden | Ash Howarth |  |

# PROJECT SUMMARY

| Report Title | Doncaster Road, Darfield – Stage 1 Road Safety Audit |
| --- | --- |
| Date: | 04/08/2026 |
| Document reference: | 2026-08 (1229J) |
| Prepared by: | Leigh Ogden |
| On behalf of: | Barnsley Council |
| AUTHORISATION SHEET |  |
| Project: | Doncaster Road, Darfield |
| Report title: | Decision Log, Doncaster Road, Darfield |
| PREPARED BY: |  |
| Name: | Leigh Ogden |
| Position: | Director |
| Signed: | LJ Ogden |
| Organisation: | Paragon Highways |
| Date: | 04/08/2026 |
| APPROVED BY: |  |
| Name: | Nathan Copley |
| Position: |  |
| Signed: |  |
| Organisation: | Barnsley Council |
| Date: |  |

# KEY PERSONNEL

| Overseeing Organisation: | Barnsley Council |
| --- | --- |
| RSA Team: | Haydn Vernals (Team Leader) & Sarah Vernals (Team Member) |
| Design organisation: | Paragon Highways |

# ROAD SAFETY AUDIT DECISION LOG

| RSA problem | RSA recommendation | Design organisation response | Overseeing Organisation response | Agreed RSA action |
| --- | --- | --- | --- | --- |
| A-01 Location: Doncaster Road – At the ghost island Summary: Drainage – Ineffective drainage of surface water may increase the risk of skidding or loss of control type collisions | It is recommended that all areas are provided with suitable surface water drainage measures to minimise the risk of standing water and detritus | Accepted – suitable drainage design shall be provided at detailed design stage and agreed with Barnsley Council as part of the S278 process |  |  |
| A-02 Location: Doncaster Road – At the ghost island junction Summary: Skidding Resistance – Carriageway surfacing across part lane widths may increase the risk of loss of control type collisions | It is recommended that the widening provided is done in full lane widths and the revised crown of the road is located such to avoid the application of any adverse camber | Accepted – the limits of the surfacing and crown position will be provided as part of the detailed design and agreed with Barnsley Council as part of the S278 process |  |  |
| A-03 Location: Barnsley Road – At the eastern development access Summary: Landscaping – Future growth from the trees and bushes may obstruct visibility consequently increasing the risk of junction related collisions | It is recommended that junction visibility envelope is cleared of vegetation | Accepted – the vegetation shall be removed insofar as to enable the formation of the junction and associated junction visibility in perpetuity |  |  |
| A-04 Location: Barnsley Road – At the eastern development access Summary: Specific Road Users – Lack of provision for users who rely on mobility aids may increase collisions involving this user group | It is recommended that dropped kerbs with tactiles are provided across the development access | Accepted – the junction will include tactile dropped crossings with further details provided at Detailed Design stage. |  |  |
| A-05 Location: Barnsley Road Summary: Pedestrians – Location of a dropped crossing may increase the risk of collisions involving pedestrians | It is recommended that the dropped crossing is located to minimise the crossing distance whilst maintaining inter-visibility between pedestrians waiting to cross and approaching vehicles. This could be achieved by relocation of the crossing points marginally further into the side road | Accepted – the tactile crossing point is located further into the site access and shown indicatively on amended drawing PRGN-1229-HGN-DR-CH-103C. The location of the crossing will be agreed with Barnsley Council as part of the S38 process |  |  |
| A-06 Location: Doncaster Road – Between the western development access and Barnsley Road Summary: Pedestrians – Close proximity to a higher speed road may increase the risk of collisions or injuries involving pedestrians on the footway | It is recommended that a buffer strip is provided appropriate to the speed of the road. Some guidance on suitable widths is highlighted in LTN 1/20 Table 6.1 for cycle routes but these remain relevant to footways | Not Accepted – the guidance relates to cycle tracks only. The proposals include a refuge island that connects to the recently constructed shared cycle facility along the north side of Doncaster Road. This contains an edge of carriageway marking providing some separation (buffer) between the carriageway and cycle facility. There are no new cycle tracks proposed on the south side of Doncaster Road. |  |  |

# DESIGN ORGANISATION AND OVERSEEING ORGANISATION STATEMENTS

| Design organisation statement |  |
| --- | --- |
| On behalf of the design organisation I certify that: 1) the RSA actions identified in response to the road safety audit problems in this road safety audit have been discussed and agreed with the Overseeing Organisation. |  |
| Name: | Leigh Ogden |
| Signed: | LJ Ogden |
| Position: | Director |
| Organisation: | Paragon Highways |
| Date: | 04/08/2026 |

| Overseeing Organisation statement |  |
| --- | --- |
| On behalf of the Overseeing Organisation I certify that: 1) the RSA actions identified in response to the road safety audit problems in this road safety audit have been discussed and agreed with the design organisation; and 2) the agreed RSA actions will be progressed. |  |
| Name: | Nathan Copley |
| Signed: |  |
| Position: |  |
| Organisation: | Barnsley Council |
| Date: |  |