Goldthorpe School Tree Protection

There are 21 trees within the Goldthorpe School boundary that are to remain and must not be damaged during the demolition work. The following protection

Trees T04-T12 will be outside the demolition site area and protected by heras fencing

Also an inner line of heras fencing will protect Trees T17-T21 going diagonal across from the Asda boundary side to building 3

Trees T14-16 will be protected by retaining the inner boundary wall as stated on the drawing NPS-DR-A-(00)-010 with additional protection as required using heras fencing with any work ( tarmac removal etc.) in the area carried out by a mini digger to minimise any damage

This just leaves the three Trees (T01 T02 T03) on the Doncaster Road these will be protected by the boundary fence, monarflex scaffold and additional heras fencing .As these trees are close to building 2 when this phase of demolition is being carried out the areas adjacent the trees will be hand demolished to minimise damage.