PLANNING CONSULTATION RESPONSE

| Application No | 2025/0831 |
| --- | --- |
| Proposal | Change of use and conversion of vacant village club (Use Class F1e) to three residential units (Use Class C3). Including demolition of existing single storey extension. |
| Address | Brierley Village Club, Church Street, Brierley, Barnsley, S72 9JG |
| Date of Consultation Reply | 5th December 2025 |
| Consultee | Highways DC |

| Consultation Assessment and Justification |  |  |
| --- | --- | --- |
| [2nd Response] With reference to the amended plans, I would now comment as follows: All six parking spaces have sufficient dimensions of 2.5m x 5m and are now shown positioned within the existing tarmac surfaced parking area to the side of the building; the red line boundary position has also been clarified. As such, the proposals include adequate parking provision and ample room for vehicles to manoeuvre within the site. With regard to the site access, it is substandard in terms of width, gradient, sight lines and drainage and is therefore wholly unsuitable for any intensification of use. It therefore comes down to a matter of determining whether or not the proposed residential use would generate a greater number of vehicular movements than the existing use. The typical number of daily two-way trips for a residential dwelling is 9 which equates to 27 trips for the three proposed dwellings. The typical number of daily two-way trips per 100m² of GFA for a village/community hall is 8 which equates to 28 trips for the existing building with a GFA of 350m². This would therefore indicate that the proposals would not constitute an intensification of use of the site. In view of the above, I do not wish to raise an objection from a highways development control perspective subject to the conditions and informative note listed below, which I would be grateful if you could include if you are minded to grant permission. |  |  |
| NO OBJECTION |  |  |
| Consultation Suggested Conditions: The parking/manoeuvring facilities, indicated on the submitted plan, shall be made available for the manoeuvring and parking of vehicles prior to the development being brought into use, and shall be retained for that sole purpose at all times. Reason: To ensure that there are adequate parking facilities to serve the development in the interests of highway safety and in accordance with Local Plan Policy T4 New Development and Transport Safety. No works shall commence on site until a scheme for the parking of bicycles complying with LTN 1/20 Cycle Infrastructure Design has been submitted to and approved in writing by the Local Planning Authority. The scheme shall be fully implemented before the development is first occupied (or brought into use) and thereafter retained for this purpose. Reason: In the interests of encouraging the use of sustainable modes of transport in accordance with Local Plan Policy T3. |  |  |
| Consultation Informative: The applicant/contractor should note that to deposit mud/debris on the public highway, or anything which may cause a nuisance or possible danger to road users, is an offence under provisions of the Highways Act 1980. |  |  |
| Planning Obligations required: – |  |  |