PLANNING CONSULTATION RESPONSE

| Application No | 2025/0907 |
| --- | --- |
| Proposal | Erection of boundary fence and new vehicle restraint system. |
| Address | Digital Media Centre, County Way, Barnsley, S70 2JW |
| Date of Consultation Reply | 6th November 2025 |
| Consultee | Highways DC |

| Consultation Assessment and Justification |  |  |
| --- | --- | --- |
| Further information and clarification is required: It is unclear where the existing boundary fence is positioned in relation to the new boundary fence; it would be useful to have the existing boundary fence shown on the proposed site plan indicated by an additional coloured dashed line. If the existing boundary fence is to be removed, details should be provided in terms of how this would be achieved without compromising the structural stability of the top of the retaining wall. The foundations for each of the boundary fence posts are proposed to be 700mm deep; details should therefore be provided in terms of the possible effect that the excavation works could have on the structural integrity of the retaining wall. The submitted drawing ‘Fencing details 2’ shows a post for the armco barrier positioned flush against the concrete foundation of the boundary fence. It is assumed that this is only indicative and that there would be no actual occurrences of the barrier posts directly abutting the foundations of the boundary fence posts. I am aware that the post spacings are different and that the distance between the fence and barrier varies but I would be grateful if clarification/confirmation could be provided. Should the applicant wish to address the above-raised issues, I shall be happy to provide further comments as necessary. |  |  |
|  | Defer for amends/further information |  |
| Consultation Suggested Conditions: |  |  |
| Consultation Informative(s): |  |  |
| Planning Obligations required: |  |  |