# Appendix F. Road safety audit response report template for stages 1, 2 and 3 only

# Fl	Project details

Provide:

Table F.1 Project details

| Report title: | PROPOSED COMMERCIAL DEVELOPMENT, HOUGHTON MAIN, BARNSLEY – PROPOSED GHOST ISLAND PRIORITY JUNCTION Combined Stage 1 and 2 Road Safety Audit |
| --- | --- |
| Date: | 21st March 2022 |
| Document reference and revision: | RSI-MH-0322-004 Issue 1.1 |
| Prepared by: | Road Safety Initiatives |
| On behalf of: | Barnsley Metropolitan Borough Council |

Table F.2 Authorisation sheet

| Project: | Houghton Main, Barnsley |
| --- | --- |
| Report title: | Stage ½ RSA Designers response |
| Prepared by: |  |
| Name: | Rik Binnersley |
| Position: | Principal Engineer |
| Signed: |  |
| Organisation: | RWO Group Leeds |
| Date: | 30.03.2022 |
| Approved by: |  |
| Name: | Andrew Fairburn |
| Position: | Director |
| Signed: |  |
| Organisation: | RWO Group Leeds |
| Date: | 30.03.2022 |

# F2	Introduction

The designers response supplements the report results from a combined Stage 1 and 2 Road Safety Audit carried out by Road Safety Initiatives on the proposed ghost island priority junction for the commercial development at Houghton Main, Barnsley scheme. Undertaken at the request of Wayne Lake, Barnsley Metropolitan Borough

The designers response was produced by Rik Binnersley of the RWO Group Leeds, the Principal Designer who was involved in the design of the S278 improvement scheme.

# F3	Key personnel

Provide:

Table F.3 Key personnel

| Overseeing Organisation: | Barnsley Metropolitan Borough Council |
| --- | --- |
| RSA team: | Mark Hedley, IEng, MCIHT, MSoRSA, FIHE, RegRSA(IHE), Alan Bawn, BSc (Hons), CEng, MICE - Road Safety Initiatives |
| Design organisation: | RWO Group, Leeds |

# F4	Road safety audit decision log

Table F.4 Road safety audit decision log

| RSA problem | RSA recommendation | Design organisation response | Overseeing Organisation response | Agreed RSA action |
| --- | --- | --- | --- | --- |
| 3.1.1 - Inadequate drainage may lead to loss of control collisions. It would appear that no drainage is to be provided adjacent to the new kerbline (currently consists of informal ’over the edge’ type drainage). The current system results in standing water, which was evident during the site visit. The carriageway is proposed to be widened, which will increase the amount of surface water to be removed and there will be nowhere for it to disperse, which could exacerbate the situation. | It is recommended that adequate drainage is provided. | Agree – At the time of submission of the drawings a drainage scheme for the works was not available. Since this time a preliminary drainage scheme has been produced (Y21011-D1008 – Section 278 Highway Drainage Layout) highlighting how the scheme will be drained. |  |  |
| 3.2.1 – Inappropriate traffic speeds and junction awareness may lead to collisions. The A6195 is subject to the national speed limit and the scheme proposes the introduction of a ghost-island junction and it appears that there is no road lighting proposed. If driver approach speeds are inappropriate and/or drivers do not appreciate the junction layout, it may lead to heavy braking incidents such as loss of control or vehicle to vehicle collisions, particularly during darkness and inclement weather conditions. Furthermore, significant speed differentials between vehicles may lead to injudicious overtaking movements. It is acknowledged that pedestrian refuges are proposed with beacons and ‘keep left’ signs which may reduce overtaking movements. However, these may not be sufficient especially during darkness and inclement weather conditions, noting that bollards will get dirty over time and at times the beacons may not be working (and no road lighting). This could result in head-on collisions or loss of control following pedestrian refuge island strikes. | It is recommended that measures to encourage appropriate traffic speeds and junction awareness are introduced. For example, - Road Lighting; - Warning lines on the approaches; - Relocate the bifurcation arrows (start of diverges in accordance with TSM Ch. 5); - Provide ‘junction’ warning signs or directional signs; - Provide a double white line system; - Road studs (edge and warning lines); and - High level ‘keep left’ signs on the beacons. | Agree – It is intended that a full street lighting scheme will be designed and submitted to BMBC for approval covering the full extent of the S278 works. The bifurcation arrows have been updated in a manner appropriate to the design speed of the highway. Road Studs have been added in a manner in keeping with the wider highway network. High Level keep left signs have been added to the island beacons. Further signage has been added to the scheme inclusive of advance warning signs and directional signs. |  |  |
| 3.3.1 – Pedestrians may be struck by a passing vehicle. The pedestrian crossing points are within lengths of hatched road markings and marginal strips. A pedestrian may choose to wait within the hatching or marginal strips and could be struck by a passing vehicle. | It is recommended that LOOK LEFT and LOOK RIGHT road markings are provided in the carriageway adjacent to the kerb lines. | Agree – Look Left and Look Right road markings added at crossing points. |  |  |
| 3.3.2 – Pedestrians may be struck by a vehicle. There is no pedestrian crossing point proposed at the development access. Pedestrians may cross the access road injudiciously and may be struck by a vehicle. Furthermore, vulnerable pedestrians such as partially sighted pedestrians, wheelchair users etc, could be at greater risk. | It is recommended that a pedestrian crossing (including tactile paving) is provided. | Disagree – Due to the width of the junction coupled with the fact that the double give way line and right turn lane would make the crossing difficult to negotiate by pedestrians, crossing of the highway by pedestrians in this location should not be promoted. There is no desire line for pedestrians to cross the highway in this location. Adequate facilities are provided elsewhere to allow pedestrians to cross from one bus stop to the other, the same facilities also make provisions for pedestrians to access the development site from both bus stops. In addition, as there is no pedestrian thoroughfare on the development side of the scheme to allow pedestrians to access the wider area. Ample crossing facilities have been provided elsewhere to allow pedestrians to safely access the footway on the opposing side of the road to the development should they wish to travel by foot to the wider area. |  |  |
| 3.3.3 – Inadequate visibility of pedestrian may lead to collisions. Vegetation may result in inadequate inter-visibility between approaching drivers from the east and pedestrians at the proposed crossing (south-side). Pedestrians must be able to see and be seen by approaching traffic. If inadequate visibility is achieved, then there is a risk of pedestrian conflicts with approaching traffic. | It is recommended that the vegetation is removed. | Agree – Vegetation will be removed from the soft margin throughout the extent of the works. However, is should be raised that this is an existing issue which should be maintained by BMBC. Planting within the soft margin should be kept at a level that does not hinder visibility. |  |  |
| 3.3.4 – Inadequate drainage may lead to pedestrian conflicts. It would appear that no drainage is to be provided adjacent to the new kerbline (currently consists of informal ’over the edge’ type drainage). The current system results in standing water, which was evident during the site visit. The carriageway is proposed to be widened, which will increase the amount of surface water to be removed and there will be nowhere for it to disperse, which could exacerbate the situation. Drainage issues, with the potential for detritus build-up may lead to ponding in the pedestrian crossing area. This may result in pedestrians crossing at inappropriate locations and increase risk of pedestrian collisions. In addition, during wintry conditions it may lead to pedestrian slips and falls. | It is recommended that adequate drainage is provided. | Agree – At the time of submission of the drawings a drainage scheme for the works was not available. Since this time a preliminary drainage scheme has been produced (Y21011-D1008 – Section 278 Highway Drainage Layout) highlighting how the scheme will be drained. However, it should be noted that the existing ponding issues seem to be caused by vegetation growing within the dished channels intended to convey water to the existing highway gullies. Upkeep of these channels to aid free draining of the existing scheme should be undertaken by BMBC. |  |  |
| 3.4.1 – Signposts may be a hazard. It is proposed to provide’ NEW ROAD LAYOUT’ and ’Pedestrian crossing’ traffic signs however, the mounting details and specific locations are not provided. These signposts may be located too close to the edge of carriageway or posts sizes may be inappropriate. Furthermore, the signs may not have suitable forward visibility if masked by vegetation. If signposts are struck they may present a hazard and subsequent loss of control or increased injury severity to vehicle occupants in an errant vehicle. Heavy braking and subsequent shunt or loss of control collisions may occur if signs do not have adeqaute visibility. In addition, adequate warning of the hazards may not be provided which could exacberbate Problem 3.2.1. | It is recommended that appropriate traffic signposts are provided. | Agree – Signage mounting heights and locations have been added to the signage drawing Y21011-D1006 - Section 278 Works To Park Spring Road Signage Arrangement. |  |  |
| 3.4.2 – Traffic signs may be a hazard for pedestrians. It is proposed to provide traffic signs throughout the scheme extents however, the mounting details are not provided. These signposts may be located within footways and restrict walkways. Signposts may be a hazard for pedestrians if located within the path, particularly for the visually impaired. Alternatively, if the posts form an obstruction, pedestrians may attempt to enter the carriageway with a risk of being struck by a passing vehicle. This issue may be exacerbated for wheelchair users. | It is recommended that the traffic signs do not obstruct pedestrians. | Agree - Signage mounting heights and locations have been added to the signage drawing Y21011-D1006 - Section 278 Works To Park Spring Road Signage Arrangement. |  |  |

# FS	Design organisation and Overseeing Organisation statements

Include the following statements to be signed by the design organisation and the Overseeing Organisation.

Table F.5 Design organisation statement

| On behalf of the design organisation I certify that: 1) the RSA actions identified in response to the road safety audit problems in this road safety audit have been discussed and agreed with the Overseeing Organisation. |  |
| --- | --- |
| Name: | Rik Binnersley |
| Signed |  |
| Position: | Principal Engineer |
| Organisation: | RWO Group Leeds |
| Date: | 30.03.2022 |

Table F.6 Overseeing Organisation statement

| On behalf of the Overseeing Organisation I certify that: - the RSA actions identified in response to the road safety audit problems in this road safety audit have been discussed and agreed with the design Organisation; and - the agreed RSA actions will be progressed. |  |
| --- | --- |
| Name: |  |
| Signed: |  |
| Position: |  |
| Organisation: |  |
| Date: |  |