PLANNING CONSULTATION RESPONSE

| Application No | 2026/0640 |
| --- | --- |
| Proposal | Residential development of 270 new dwellings, together with access, public open space, landscaping, drainage infrastructure and associated works. |
| Address | Land south of Cemetery Road, Jump, Barnsley, S73 0QN. |
| Date of Consultation Reply | 23/10/2026 |
| Consultee | Edward Jowett (Tree Officer) |

| Consultation Assessment and Justification |  |  |
| --- | --- | --- |
| The trees and hedges on site are restricted to the existing field boundaries. The hedges are generally complete with few gaps and in good condition. The trees are located just on or over the external boundaries of the site with the exception of two Ash trees which are located at the end of a hedge towards the southern end of the site referenced as T17 and T18. The scheme has largely been designed around the trees and hedges allowing all trees and much of the hedgerows to be retained. There are however multiple breaks in the hedgerows, in the main to facilitate the access roads through the scheme. Whilst in strictly arboricultural terms this is just about acceptable I would note that in terms of Biodiversity this may not be the case. The majority of the trees and the retained hedgerows have been incorporated into the scheme well with sufficient clearance from dwellings and other built features being provided although in instances such as plot 4, the dwelling is quite close to H003. There are works close to T17 and T18, however these encroachments into the trees rooting areas are very minor and well with in the limits of what can be tolerated by the trees. Ideally the trees would remain unaffected by the works but as detailed in the arboricultural impact assessment and arboricultural method statement, the works are specified in such a manner as to be sympathetic to the trees to minimise the impact and coupled with the small percentage of their rooting area which will be affected these should be easily tolerated by them. Given that some sections of hedgerow are to be removed I would note that significant new planting will be required. Ideally new native hedges would be incorporated into the scheme to mitigate for the loss of hedgerows along with new tree planting. A landscape masterplan has been provided which shows significant new planting and new native hedgerow, however at this stage we do not appear to have the details of the full planting specifications and species to be used etc. and as such this will need to be conditioned so it can be ensured that suitable species are selected for the right places. |  |  |
| NO OBJECTION |  |  |
| *Delete as applicable |  |  |
| Consultation Suggested Conditions: No development shall take place until there has been submitted to and approved in writing by the Local Planning Authority, full details of both hard and soft landscaping works, including details of the species, positions and planted heights of proposed trees and shrubs; together with details of the position and condition of any existing trees and hedgerows to be retained. The approved landscaping details shall be implemented prior to the occupation of the first dwelling. Reason: In the interests of the visual amenities of the locality. |  |  |
| Consultation Informative(s): N/A |  |  |
| Planning Obligations required: N/A |  |  |