IMPACT ON INDIVIDUAL TREES

ALONG DESIGNED SECTIONS OF ACCESS ROAD

Prepared by :-

David C Houldershaw

Northern Resource Consultants

22 Ethel Crescent

Knaresborough

North Yorkshire

HG5 0DJ

01423 868952

treelawexpert@aol.com

arboriculture, forestry, ecology, landscape & the environment

Over 43 years experience in planning & litigation involving trees & wildlife

September 2024

SECTION A – INTRODUCTION

A1.0	Introduction

A1.1	This report supplements my previous submission looking more broadly at the potential arboricultural impacts of the proposed allocated land development.

A1.2	Details of the individual trees are provided in the Appendix A Tree Schedule.

A1.3	The Table in Section B considers the impact on individual trees and outlines potential mitigation, the details of which can be agreed with the Council at Reserved Matters stage including detailed AIA Method Statement, arboricultural monitoring and supervision.

A1.4	Tree numbers as per recent tagging.

A2.0	Summary

A2.1	In the southern access area :-

3 no. trees require removal for unrelated safety reasons :- T1 Ash, T3 Elm, tree C Horse chestnut

c. 30 no. trees in 8 elements require removal for the road and retaining wall :- Sycamore rw1, 2 untagged (u/t) Ash, two stretches of u/t Cypress hedge, Ash rw2, most of group G2 east of existing footpath, groups G3 and G4; many of the trees are small and easily replaced and replacement on a two-for-one basis is proposed (and this can be increased)

A2.2	In the southern access area, the road works impinge on the root protection areas of T4 Oak inside the site and Sycamores A and B outside the site; there is potential for T4 and B to recover, A is already in visible decline; if T4 fails to fully recover, the tree can be veteranized and retained for habitat; and potential replacements provided for each; as the detail for the site is further worked for Reserved Matters, the retaining wall in the vicinity of Sycamore B can be studied to see if there is any further means of easing away from the tree,

A2.3	In the central woodland area :-

u/t dead and u/t Oak require removal for unrelated safety reasons

11 no. trees require removal for the road and retaining wall ;- Sycamores cw6, cw22, cw23, cw31, Hawthorns cw7, cw8, cw17 to cw21 inclusive, Oaks cw29, cw30, cw31 together with minor understorey; many of the trees are small and easily replaced and replacement on a two-for-one basis is proposed (and this can be increased)

A2.4	In the central woodland area, the road works impinge on the root protection areas of Oak cw4, Willows cw5, cw9, cw10, and Hazel cw11; this ism mostly minor and recoverable but if any trees fail to recover, replacements can be planted as allowed for in the allocation.

SECTION B – ARBORICULTURAL IMPACT ASSESSMENT

See table overleaf

| T | SPECIES | CAT | RPA | DEVELOPMENT DISTANCE | MITIGATION MEASURES | IMPACT ON TREE |
| --- | --- | --- | --- | --- | --- | --- |
| T1 | Ash | U | 3.7 | 5 m to retaining wall | Felling recommended for unrelated safety reasons; retain coppice growth as replacement | No additional impact from development |
| T2 | Ash | B/C | 6.0 | 8 m to retaining wall | Retain and protect 1) Erect tree protection at perimeter of RPA; BS5837:2012 default specification min 2 m high metal weld mesh fencing on scaffold frame fully braced on tree side against movement; warning notices as per BS; to exclude construction ingress 2} Erect fencing before any site works begin; project Arboriculturist to inspect, notify Council and supply photographic record | No works in RPA; RPA fully protected with robust fencing as per BS to avoid damage No adverse impact from development as proposed |
| T3 | Elm | U | 2.2 | 7.5 m to retaining wall | Felling recommended for unrelated safety reasons, retain coppice growth as replacement | No additional impact from development |
| T4 | Oak | B | 13.2 | 7.5 m to retaining wall | Retain and protect 1) Erect tree protection fencing at RPA at distance from tree; tree protection fencing at 1 m from retaining wall where inside RPA; BS default specification as per Ash T2 1) above 2) Lay ground protection matting between tree protection fencing and retaining wall to allow access for wall construction; to be laid on compressible bark layer as per BS and adequate to avoid compaction 3) Erect fencing, lay ground protection, inspect and notify Council before any site works begin as per Ash T2 2) above 4) Before any site works begin; works to reduce and veteranize the tree to be agreed with the Council and implemented 5) Project Arboriculturist to monitor and supervise road and wall works in RPA | Retaining wall and road in RPA Works outside root plate and unlikely to destabilise tree; modest reduction in height of 3 m should be sufficient to reduce any additional risk of windblow to present (normal) levels At sufficient distance where if no other works in RPA it is possible for a healthy reasonably vigorous tree such as this to survive and recover thereby justifying retention If, however, the tree fails to fully recover, management can be undertaken to accelerate veteran characteristics thereby justifying retention as habitat If the site is to be developed for housing as allocated, access through the RPA is unavoidable; and - in the very specific circumstances here - on balance - the impact on the tree is acceptable All works in RPA under arboricultural supervision to minimise damage |
| T5 | Ash | B/ C | 3.8 | 5.5 m to retaining wall | Retain and protect 1) Tree protection fencing at perimeter of RPA all as per Ash T2 1) and 2) above | No works in RPA; RPA fully protected with robust fencing as per BS to avoid damage No adverse impact from development as proposed |
|  | Mixed trees |  |  | Outside site on west side of stream with no rooting east of stream into site area | Retain; protected by distance and the stream 1) Usual site boundary fencing adequate to prevent construction access with no additional precautions required | No works in RPA; outside site boundary; RPA fully protected by steep slope and stream supplemented by the usual site safety and security fencing erected along the site boundary No adverse impact from development as proposed |
| u/t | Oak | B | 8.4 | 11 m to retaining wall | Retain and protect 1) Tree protection fencing at perimeter of RPA all as per Ash T2 1) and 2) above | No works in RPA; RPA fully protected with robust fencing as per BS to avoid damage No adverse impact from development as proposed |
| G1 | Mixed group west of path |  | Up to 3.1 | >6 m to retaining wall | Retain and protect 1} Tree protection fencing at perimeter of RPA all as per Ash T2 1) and 2) above | No works in RPA; RPA fully protected with robust fencing as per BS to avoid damage No adverse impact from development as proposed |
| G2 | Mixed group east of path |  | Up to 2.6 | 0 to >4 m from retaining wall | Part remove for road and retaining wall; part retain and protect 1) Retain where road and retaining wall outside RPA and erect tree protection fencing at perimeter of RPA as per Ash T2 1) and 2) above 2) Fell where works inside RPA; replace on a two-for-one basis in site preferably along the access road in areas contiguous with the woodland | Road and retaining wall partly in RPA Where works outside RPA; RPA fully protected with robust fencing as per BS to avoid damage Where works inside RPA, fell and replace as per the specific requirements of the allocation – allocation requires any trees removed to be replaced Trees to be removed are small and relatively easily replaced with short term impact on amenity; planting contiguous to woodland restores woodland area As removed trees are replaced and there is no net loss of woodland area there is no adverse impact from development as proposed |
| rw1 | Sycamore | C | 3.6 | 1.5 m retaining wall | Remove for retaining wall 1) Plant 2 no. Sycamores as replacements preferably along the access road in areas contiguous with the woodland | Road and retaining wall in RPA Replacement required by the allocation; proposed replacement is two-for-one For development as allocated, access through the RPA is unavoidable; replacement planting is to be undertaken and - in the very specific circumstances here - on balance - the impact is acceptable |
| u/t | Ash | C | 1.0 | <1 m retaining wall Small tree adjacent to rw1 | Remove for retaining wall 1) Plant 2 no. Common alder as replacements preferably along the access road in areas contiguous with the woodland | Road and retaining wall in RPA Replacement required by the allocation; proposed replacement is two-for-one; small tree easily replaced and the impact is acceptable not least as access is required for the allocation to be delivered |
| u/t | Cypress hedge | C | 1.8 | In retaining wall and road Short hedge between rw1 and rw2 | Remove for retaining wall and road 1) Plant new native hedging along field boundary as replacement | In retaining wall and road area Minor conifer hedging relatively easily replaced; replacement with native hedging and the impact is acceptable not least as access is required for the allocation to be delivered |
| rw2 | Ash | C/U | 7.2 | In road | Remove for road 1) Plant 2 no. Small leaf lime as replacements preferably along the access road in areas contiguous with the woodland | Road and retaining wall in RPA Replacement required by the allocation; proposed replacement is two-for-one For development as allocated, access through the RPA is unavoidable; replacement planting is to be undertaken and - in the very specific circumstances here - on balance - the impact is acceptable |
| u/t | Ash | C | 2.2 | In road Small tree adjacent to rw2 | Remove for road 1) Plant 2 no. Small leaf lime as replacements preferably along the access road in areas contiguous with the woodland | Road and retaining wall in RPA Replacement required by the allocation; proposed replacement is two-for-one; small tree easily replaced and the impact is acceptable not least as access is required for the allocation to be delivered |
| rw3 | Ash | U | 3.0 | In road | Felling recommended for unrelated safety reasons; plant replacement | No additional impact from development |
| u/t | Cypress hedge | C | 3.0 | In retaining wall and road Short hedge between rw1 and rw2 | Remove for retaining wall and road 1) Plant new native hedging along field boundary as replacement | In retaining wall and road area Minor conifer hedging relatively easily replaced; replacement with native hedging and the impact is acceptable not least as access is required for the allocation to be delivered |
| A | Sycamore | C/U | 12.0 | 5 m retaining wall Outside site and visibly in decline | Outside site; protect 1) Erect tree protection fencing at RPA at distance from tree; tree protection fencing at 1 m from retaining wall where inside RPA; BS default specification as per Ash T2 1) above 2) Lay ground protection matting between tree protection fencing and retaining wall to allow access for wall construction; to be laid on compressible bark layer as per BS and adequate to avoid compaction 3) Erect fencing, lay ground protection, inspect and notify Council before any site works begin as per Ash T2 2) above 4) Project Arboriculturist to monitor and supervise road and wall works in RPA | Retaining wall and road in RPA Works outside root plate and unlikely to destabilise tree; in decline with naturally reducing crown Works unlikely to accelerate decline but knowledge of cause would be useful to ensure safety of all including tree owner If the site is to be developed for housing as allocated, access through the RPA is unavoidable; and - in the very specific circumstances here - on balance - the impact on the tree is acceptable All works in RPA under arboricultural supervision to minimise damage |
| B | Sycamore | B | 13.2 | 8 m to pavement 3 m to retaining wall | Outside site; protect 1) Erect tree protection fencing at RPA at distance from tree; tree protection fencing at 1 m from retaining wall where inside RPA; BS default specification as per Ash T2 1) above 2) Lay ground protection matting between tree protection fencing and retaining wall to allow access for wall construction; to be laid on compressible bark layer as per BS and adequate to avoid compaction 3) Erect fencing, lay ground protection, inspect and notify Council before any site works begin as per Ash T2 2) above 4) Project Arboriculturist to monitor and supervise road and wall works in RPA | Retaining wall and road in RPA Works at edge of root plate but as end of retaining wall significant rooting unaffected by retaining wall Extent of works in this area sufficient to adversely impact many Sycamores of this size and age As the tree may go into decline, two Sycamores should be planted to act as potential replacements to meet the allocation requirements If the site is to be developed for housing as allocated, access through the RPA is unavoidable; and - in the very specific circumstances here - on balance - the impact on the tree is acceptable All works in RPA under arboricultural supervision to minimise damage |
| C | Horse chestnut | U | 14.4 | 6 m to end of retaining wall 10 m to pavement | Felling recommended for unrelated safety reasons; plant replacement | No additional impact from development |
| G3 | Ash and Elder | C | 1.8 | In road | Remove for road 1) Plant 10 no. Small leaf lime, 4 no Elder as replacements preferably along the access road in areas contiguous with the woodland | In road Replacement required by the allocation; proposed replacement is two-for-one; small tree easily replaced and the impact is acceptable not least as access is required for the allocation to be delivered |
| G4 | Ash group | C/U | 1.8 | In road | Remove three of seven for road, remove remaining four for unrelated Ash Dieback Disease and plant replacements 1) Plant 14 no. Small leaf lime as replacements for both preferably along the access road in areas contiguous with the woodland | In road Replacement required by the allocation; proposed replacement is two-for-one; small tree easily replaced and the impact is acceptable not least as access is required for the allocation to be delivered |

Central Woodland

| T | SPECIES | CAT | RPA | DEVELOPMENT DISTANCE | MITIGATION MEASURES | IMPACT ON TREE |
| --- | --- | --- | --- | --- | --- | --- |
| cw1 | Willow | C/U | 8.4 | 11 m retaining wall | Retain and prune for safety 1) Erect tree protection at perimeter of RPA; BS5837:2012 default specification min 2 m high metal weld mesh fencing on scaffold frame fully braced on tree side against movement; warning notices as per BS; to exclude construction ingress 2} Erect fencing before any site works begin; project Arboriculturist to inspect, notify Council and supply photographic record | No works in RPA; RPA fully protected with robust fencing as per BS to avoid damage No adverse impact from development as proposed |
| cw2 | Willow | B | 9.6 | 8 m to retaining wall | Retain and protect 1) Erect tree protection fencing at RPA at distance from tree; tree protection fencing at 1 m from retaining wall where inside RPA; BS default specification as per Willow cw1 1) above 2) Lay ground protection matting between tree protection fencing and retaining wall to allow access for wall construction; to be laid on compressible bark layer as per BS and adequate to avoid compaction 3) Erect fencing, lay ground protection, inspect and notify Council before any site works begin as per Willow cw1 1) above 4) Project Arboriculturist to monitor and supervise road and wall works in RPA | Retaining wall and road in RPA Works outside root plate and unlikely to destabilise tree At sufficient distance where if no other works in RPA it is possible for a healthy reasonably vigorous tree such as this to survive and recover thereby justifying retention If, however, the tree fails to fully recover as expected, replacements can be planted If the site is to be developed for housing as allocated, access through the RPA is unavoidable; and - in the very specific circumstances here - on balance - the impact on the tree is acceptable All works in RPA under arboricultural supervision to minimise damage |
| cw3 | Oak | C | 3.2 | 6 m to retaining wall | Retain and protect 1) Tree protection fencing at perimeter of RPA all as per Willow cw1 1) and 2) above | No works in RPA; RPA fully protected with robust fencing as per BS to avoid damage No adverse impact from development as proposed |
| cw4 | Oak | B | 4.8 | 4.5 m to retaining wall | Retain and protect 1) Erect tree protection fencing at RPA at distance from tree; tree protection fencing at 1 m from retaining wall where inside RPA; BS default specification as per Willow cw1 1) above 2) Lay ground protection matting between tree protection fencing and retaining wall to allow access for wall construction; to be laid on compressible bark layer as per BS and adequate to avoid compaction 3) Erect fencing, lay ground protection, inspect and notify Council before any site works begin as per Willow cw1 1) above 4) Project Arboriculturist to monitor and supervise road and wall works in RPA | Retaining wall in RPA Works outside root plate and unlikely to destabilise tree Minor incursion into RPA and should recover without detriment; if, however, the tree fails to fully recover as expected, replacements can be planted If the site is to be developed for housing as allocated, access through the RPA is unavoidable; and - in the very specific circumstances here - on balance - the impact on the tree is acceptable All works in RPA under arboricultural supervision to minimise damage |
| cw5 | Willow | C/U | 8.5 | 5 m to retaining wall | Retain and protect, prune for safety 1) Erect tree protection fencing at RPA at distance from tree; tree protection fencing at 1 m from retaining wall where inside RPA; BS default specification as per Willow cw1 1) above 2) Lay ground protection matting between tree protection fencing and retaining wall to allow access for wall construction; to be laid on compressible bark layer as per BS and adequate to avoid compaction 3) Erect fencing, lay ground protection, inspect and notify Council before any site works begin as per Willow cw1 1) above 4) Project Arboriculturist to monitor and supervise road and wall works in RPA | Retaining wall and road in RPA Works outside root plate and unlikely to destabilise tree At sufficient distance where if no other works in RPA it is possible for a healthy reasonably vigorous tree such as this to survive and recover thereby justifying retention as a habitat tree with pre-existing condition managed for path safety If, however, the tree fails to recover as expected, replacements can be planted If the site is to be developed for housing as allocated, access through the RPA is unavoidable; and - in the very specific circumstances here - on balance - the impact on the tree is acceptable All works in RPA under arboricultural supervision to minimise damage |
| cw6 | Sycamore | B/C | 5.1 | 1.5 m to retaining wall | Remove for retaining wall and road 1) Plant 2 no. Sycamores as replacements preferably along the access road in areas contiguous with the woodland | Road and retaining wall in RPA Replacement required by the allocation; proposed replacement is two-for-one For development as allocated, access through the RPA is unavoidable; replacement planting is to be undertaken and - in the very specific circumstances here - on balance - the impact is acceptable |
| cw7 | Hawthorn | C | 2.9 | In road | Remove for road 1) Plant 2 no. Hawthorns as replacements preferably along the access road in areas contiguous with the woodland | In road; small tree relatively easily replaced Replacement required by the allocation; proposed replacement is two-for-one For development as allocated, access through the RPA is unavoidable; replacement planting is to be undertaken and - in the very specific circumstances here - on balance - the impact is acceptable |
| cw8 | Hawthorn | B/C | 5.9 | 4.5 m to retaining wall | Remove for retaining wall 1) Plant 2 no. Hawthorns as replacements preferably along the access road in areas contiguous with the woodland | Retaining wall in RPA Replacement required by the allocation; proposed replacement is two-for-one For development as allocated, access through the RPA is unavoidable; replacement planting is to be undertaken and - in the very specific circumstances here - on balance - the impact is acceptable |
| cw9 | Willow | B/C | 11.4 | 6.5 m to retaining wall | Retain and protect, prune for safety 1) Erect tree protection fencing at RPA at distance from tree; tree protection fencing at 1 m from retaining wall where inside RPA; BS default specification as per Willow cw1 1) above 2) Lay ground protection matting between tree protection fencing and retaining wall to allow access for wall construction; to be laid on compressible bark layer as per BS and adequate to avoid compaction 3) Erect fencing, lay ground protection, inspect and notify Council before any site works begin as per Willow cw1 1) above 4) Project Arboriculturist to monitor and supervise road and wall works in RPA | Retaining wall and road in RPA Works outside root plate and unlikely to destabilise tree At sufficient distance where if no other works in RPA it is possible for a healthy reasonably vigorous tree such as this to survive and recover thereby justifying retention as a habitat tree with pre-existing condition managed for path safety If, however, the tree fails to recover as expected, replacements can be planted If the site is to be developed for housing as allocated, access through the RPA is unavoidable; and - in the very specific circumstances here - on balance - the impact on the tree is acceptable All works in RPA under arboricultural supervision to minimise damage |
| cw10 | Willow | C/U | 5.4 | 5 m to retaining wall Collapsed and prone tree | Retain and protect; prune for path safety 1) Tree protection fencing at perimeter of RPA all as per Willow cw1 1) and 2) above | Minor incursion into RPA; RPA mostly fully protected with robust fencing as per BS to avoid damage No adverse impact from development as proposed |
| cw11 | Hazel | B/C | 4.8 | 4 m to retaining wall | Retain and protect 1) Erect tree protection fencing at RPA at distance from tree; tree protection fencing at 1 m from retaining wall where inside RPA; BS default specification as per Willow cw1 1) above 2) Lay ground protection matting between tree protection fencing and retaining wall to allow access for wall construction; to be laid on compressible bark layer as per BS and adequate to avoid compaction 3) Erect fencing, lay ground protection, inspect and notify Council before any site works begin as per Willow cw1 1) above 4) Project Arboriculturist to monitor and supervise road and wall works in RPA | Retaining wall and road in RPA Works outside root plate and unlikely to destabilise At sufficient distance where if no other works in RPA it is possible for a healthy reasonably vigorous shrub such as this to survive and recover If fails to recover as expected, replacements can be planted If the site is to be developed for housing as allocated, access through the RPA is unavoidable; and - in the very specific circumstances here - on balance - the impact is acceptable All works in RPA under arboricultural supervision to minimise damage |
| cw12 | Hawthorn | C | 1.9 | 3 m to retaining wall | Retain and protect 1) Tree protection fencing at perimeter of RPA all as per Willow cw1 1) and 2) above | No works in RPA; RPA fully protected with robust fencing as per BS to avoid damage No adverse impact from development as proposed |
| cw13 | Willow | C | 4.9 | 5 m to retaining wall | Retain and protect 1) Tree protection fencing at perimeter of RPA all as per Willow cw1 1) and 2) above | No works in RPA; RPA fully protected with robust fencing as per BS to avoid damage No adverse impact from development as proposed |
| cw14 | Hawthorn | C | 3.0 | 3.5 m to retaining wall | Retain and protect 1) Tree protection fencing at perimeter of RPA all as per Willow cw1 1) and 2) above | No works in RPA; RPA fully protected with robust fencing as per BS to avoid damage No adverse impact from development as proposed |
| cw15 | Hawthorn | C | 2.4 | 3 m to retaining wall | Retain and protect 1) Tree protection fencing at perimeter of RPA all as per Willow cw1 1) and 2) above | No works in RPA; RPA fully protected with robust fencing as per BS to avoid damage No adverse impact from development as proposed |
| cw16 | Elder group | C | 1.8 | In road | Remove for road 1) Plant Elders as replacement | In road; easily replaced Minor understorey shrubbery relatively easily replaced; replacement on a two-for-one basis; and the impact is acceptable not least as access is required for the allocation to be delivered |
| cw17 | Hawthorn | B | 6.0 | In road | Remove for road 1) Plant 2 no. Hawthorns as replacements preferably along the access road in areas contiguous with the woodland | In road Replacement required by the allocation; proposed replacement is two-for-one For development as allocated, access through the RPA is unavoidable; replacement planting is to be undertaken and - in the very specific circumstances here - on balance - the impact is acceptable |
| cw18 | Hawthorn | B/C | 2.4 | In road | Remove for road 1) Plant 2 no. Hawthorns as replacements preferably along the access road in areas contiguous with the woodland | In road; easily replaced Replacement required by the allocation; proposed replacement is two-for-one For development as allocated, access through the RPA is unavoidable; replacement planting is to be undertaken and - in the very specific circumstances here - on balance - the impact is acceptable |
| cw19 | Hawthorn | C | 2.4 | In road | Remove for road 1) Plant 2 no. Hawthorns as replacements preferably along the access road in areas contiguous with the woodland | In road; easily replaced Replacement required by the allocation; proposed replacement is two-for-one For development as allocated, access through the RPA is unavoidable; replacement planting is to be undertaken and - in the very specific circumstances here - on balance - the impact is acceptable |
| cw20 | Hawthorn | C | 2.1 | In road | Remove for road 1) Plant 2 no. Hawthorns as replacements preferably along the access road in areas contiguous with the woodland | In road; easily replaced Replacement required by the allocation; proposed replacement is two-for-one For development as allocated, access through the RPA is unavoidable; replacement planting is to be undertaken and - in the very specific circumstances here - on balance - the impact is acceptable |
| cw21 | Hawthorn | C | 3.0 | In road | Remove for road 1) Plant 2 no. Hawthorns as replacements preferably along the access road in areas contiguous with the woodland | In road; easily replaced Replacement required by the allocation; proposed replacement is two-for-one For development as allocated, access through the RPA is unavoidable; replacement planting is to be undertaken and - in the very specific circumstances here - on balance - the impact is acceptable |
| cw22 | Sycamore | B/C | 3.6 | In road | Remove for road 1) Plant 2 no. Sycamores as replacements preferably along the access road in areas contiguous with the woodland | In road; relatively easily replaced Replacement required by the allocation; proposed replacement is two-for-one For development as allocated, access through the RPA is unavoidable; replacement planting is to be undertaken and - in the very specific circumstances here - on balance - the impact is acceptable |
| cw23 | Sycamore | C | 3.4 | 1.5 m to pavement | Remove for road 1) Plant 2 no. Sycamores as replacements preferably along the access road in areas contiguous with the woodland | In road; easily replaced Replacement required by the allocation; proposed replacement is two-for-one For development as allocated, access through the RPA is unavoidable; replacement planting is to be undertaken and - in the very specific circumstances here - on balance - the impact is acceptable |
| cw24 | Sycamore | B | 8.9 | 16 m road | Retain and protect 1) Tree protection fencing at perimeter of RPA all as per Willow cw1 1) and 2) above | No works in RPA; RPA fully protected with robust fencing as per BS to avoid damage No adverse impact from development as proposed |
| cw25 | Oak | B/C | 4.8 | 12 m to pavement | Retain and protect 1) Tree protection fencing at perimeter of RPA all as per Willow cw1 1) and 2) above | No works in RPA; RPA fully protected with robust fencing as per BS to avoid damage No adverse impact from development as proposed |
| cw26 | Oak | B | 4.6 | 13 m to pavement | Retain and protect 1) Tree protection fencing at perimeter of RPA all as per Willow cw1 1) and 2) above | No works in RPA; RPA fully protected with robust fencing as per BS to avoid damage No adverse impact from development as proposed |
| cw27 | Sycamore | B | 4.4 | 7.5 m to pavement | Retain and protect 1) Tree protection fencing at perimeter of RPA all as per Willow cw1 1) and 2) above | No works in RPA; RPA fully protected with robust fencing as per BS to avoid damage No adverse impact from development as proposed |
| cw28 | Oak | B | 4.0 | 6 m pavement | Retain and protect 1) Tree protection fencing at perimeter of RPA all as per Willow cw1 1) and 2) above | No works in RPA; RPA fully protected with robust fencing as per BS to avoid damage No adverse impact from development as proposed |
| cw29 | Oak | B | 5.4 | 3 m to retaining wall | Remove for road 1) Plant 2 no. Oaks as replacements preferably along the access road in areas contiguous with the woodland | Retaining wall in RPA Replacement required by the allocation; proposed replacement is two-for-one For development as allocated, access through the RPA is unavoidable; replacement planting is to be undertaken and - in the very specific circumstances here - on balance - the impact is acceptable |
| cw30 | Oak | B | 4.8 | In road area | Remove for road 1) Plant 2 no. Oaks as replacements preferably along the access road in areas contiguous with the woodland | In road; reasonably easily replaced Replacement required by the allocation; proposed replacement is two-for-one For development as allocated, access through the RPA is unavoidable; replacement planting is to be undertaken and - in the very specific circumstances here - on balance - the impact is acceptable |
| u/t | Mixed group |  | Up to 1.44 | In road | Remove for road 1) Plant 2 no. Oaks as replacements preferably along the access road in areas contiguous with the woodland | In road; reasonably easily replaced Replacement required by the allocation; proposed replacement is two-for-one For development as allocated, access through the RPA is unavoidable; replacement planting is to be undertaken and - in the very specific circumstances here - on balance - the impact is acceptable |
| cw31 | Oak | C | 3.0 | 1.5 m to pavement | Remove for road 1) Plant 2 no. Oaks as replacements preferably along the access road in areas contiguous with the woodland | In road; easily replaced Replacement required by the allocation; proposed replacement is two-for-one For development as allocated, access through the RPA is unavoidable; replacement planting is to be undertaken and - in the very specific circumstances here - on balance - the impact is acceptable |
| cw32 | Sycamore | B | 4.8 | In road area | Remove for road 1) Plant 2 no. Sycamores as replacements preferably along the access road in areas contiguous with the woodland | In road; easily replaced Replacement required by the allocation; proposed replacement is two-for-one For development as allocated, access through the RPA is unavoidable; replacement planting is to be undertaken and - in the very specific circumstances here - on balance - the impact is acceptable |
| u/t | Dead | U | n/a |  | Remove for unrelated safety reasons; plant replacement | No additional impact resulting from development |
| u/t | Oak | U |  |  | Remove for unrelated safety reasons; plant replacement | No additional impact resulting from development |
| cw33 | Oak | C | 3.0 | 3 m to pavement | Retain and protect 1) Tree protection fencing at perimeter of RPA all as per Willow cw1 1) and 2) above | No works in RPA; RPA fully protected with robust fencing as per BS to avoid damage No adverse impact from development as proposed |

TREE SCHEDULE

KEY

TREE				Tree number or Code letter

SPECIES			Common name

D				Diameter at breast height in cm

SP				Spread in m at the four cardinal points

HT				Overall height in m

AGE				Age stage :- 	Y, young; EM, early maturity; M, mature;

LM; late maturity; S, Senile; D, Dead

LIFE				Estimate of remaining useful safe life in years

CAT				BS5837:2012 tree quality assessment category

RPA				BS5837:2012 radial root protection area distance

DESCRIPTION			Brief description

RECOMMENDATIONS		Recommended work in existing use

COMMENTS			Any further comments

Notes

1. Latin names omitted for brevity.

Southern Access

| NO | SPECIES | D | SP | HT | AGE | LIFE | CAT | RPA | DESCRIPTION | RECOMMENDATIONS | COMMENTS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T1 | Ash | 23+ 18+ 10 | N 2 E 6 S 6 W 5 | 10 | EM | <10 | U | 3.7 | Young tree; forked at base into 3; growth towards road; main stems with longitudinal damage and internal decay developing; under threat from Ash Dieback Disease | Fell (for safety) | Risk increasing with extension growth of significant breakage onto road and pavement (T3 on Quants) |
| T2 | Ash | 55 | N 7 E 8 S 8 W 8 | 21 | M | 30+ or <10 | B/C | 6.0 | Maturing tree; dense ivy prevents full inspection; lower dead branch over path; splitting branch on south-west side over pavement and road; under threat from Ash Dieback Disease | Remove dead branch over path; reduce splitting branch over pavement and road (both for safety) | (T1 on Quants) |
| T3 | Elm | 18 |  | 8 | S | <5 | U | 2.2 | Small suppressed tree; significant Dutch Elm Disease; virtually dead | Make safe and retain for habitat | Virtually dead; easily repl;aced |
| T4 | Oak | 110 | N 6 E 9 S 10 W10 | 19 | LM | 30+ | B | 13.2 | Large heavy mature tree; past bough breakage; heavy dead wood in crown; cavities in crown | Climbing inspection for splits, cavities, not visible or assessable from ground | T4 and T5 transposed on plans in respect of size; T4 is the larger tree (T4 on Quants) |
| T5 | Ash | 32 | N 3 E 3 S 3 W 3 | 16 | M | 30+ or <10 | B/ C | 3.8 | Smaller mature tree, under threat from Ash Dieback Disease | No work at present | Not separately recorded in Quants |
|  | Mixed trees |  |  |  |  |  |  |  | Group of mostly mature trees on west side of stream and separated from site works by the stream | Outside site area | On opposite side of stream to access and other site works; stream is a barrier to rooting with no rooting or RPA on site side of stream |
| u/t | Oak | 70 | N 8 E 2 S 0 W 8 | 16 | M | 30+ | B | 8.4 | Mature tree on east side of stream near bottom of steep slope | No work at present | Not separately recorded in Quants |
| G1 | Mixed group west of path |  |  |  |  |  |  | Up to 3.1 | Group of young trees west of existing path including Wild cherry 26cm Dbh, 12 m height (largest tree in group), 1 Sycamore 10 cm Dbh, 7 m height, 8 Oak saplings <8 cm Dbh, 2 Wild cherry saplings <7 cm Dbh, elder | Thin to favour best (to create adequate growing space) | (Wild cherry, 26 cm Dbh, is T9 in Quants) group is G4 in Quants; easily replaced |
| G2 | Mixed group east of path |  |  |  |  |  |  | Up to 2.6 | Group of young trees including short remnant of old Hawthorn hedge <3 m height, 4 young Ash largest 22 cm, 13 m height, 2 Goat willow largest 15 cm, 9 m height, 1 Rowan 15 cm, 8 m height, 3 Cherry largest 30 cm, 11 m height and several Cherry laurel | Thin to favour best (to create adequate growing space) | Group is G2 in Quants; easily replaced |
| rw1 | Sycamore | 30 | N 3 E 4 S 5 W 4 | 15 | EM | 30+ | C | 3.6 | Young tree; grows on top of drystone retaining wall and gradually causing collapse | Monitor stability | Minor risk of instability; monitor (included in Quants as G1) |
| u/t | Ash | 8 |  | 7 | Y | <10 or <30 | C | 1.0 | Untagged young tree/ large sapling; suppressed; at risk of Ash Dieback Disease | No work at present | Very little long term potential (included in Quants as G1); easily replaced |
| u/t | Cypress hedge | <15 |  | 7 | EM | 30 | C | 1.8 | Short stretch of overgrown Cypress hedge at top of retaining wall and causing damage; suppressed | Monitor stability | Minor risk of instability; monitor (included in Quants as G1); easily replaced |
| rw2 | Ash | 60 | N 5 E 6 S 6 W 5 | 18 | LM | <10 | C/U | 7.2 | Mature tree; at top of retaining wall and causing damage; thinning crown, recently dead branches and unflushed twigs; in early stages of Ash Dieback | Remove heavy dead branches (for safety) | Early stages of decline; decline requires managing as over footpath (T5 in Quants) |
| u/t | Ash | 18 |  | 7 | EM | <20 or <10 | C | 2.2 | Young Ash, crowded and suppressed; at risk of Ash Dieback Disease |  | (included in Quants as G1); easily replaced |
| rw3 | Ash | 25 | N 6 E 3 S 2 W 2 | 10 | EM | <5 | U | 3.0 | Young Ash; Inonotus hispidus bracket on north side with increasing risk of significant stem breakage | Fell (for path safety) | Serious decay pathogen of Ash, the most common cause of stem breakage (included in Quants as G1) |
| u/t | Cypress hedge | <25 |  | 4 | EM | 30 | C | 3.0 | Short stretch of overgrown Cypress hedge at top of retaining wall and causing damage; suppressed | Monitor stability | Minor risk of instability; monitor (included in Quants as G1) |
| A | Sycamore | 100 | N 2 E 9 S 8 W 8 | 22 | LM/S | <20 | C/U | 12.0 | Large heavy mature tree; significant crown dieback; heavy dead wood in crown; not inspected for safety as part of this survey | Outside ownership | Outside ownership; dieback potentially indicative of serious root or stem disease and/ or decay (T6 in Quants) |
| B | Sycamore | 110N | N 5 E 10 S 8 W10 | 22 | LM | 30+ | B | 13.2 | Large heavy mature tree; not inspected for safety as part of this survey | Outside ownership | Outside ownership (T7 in Quants) |
| C | Horse chestnut | 120 | N 10 E 9 S 5 W 5 | 22 | LM/S | 0 | U | 14.4 | Large heavy mature tree; significant basal decay; well below usual tolerable safety parameters | Fell (for path safety) (urgent) | Increasingly very likely to collapse or break including towards path or into garden (T8 in Quants) |
| G3 | Ash and Elder | 15 | N 2 E 2 S 2 W 2 | 8 | Y | 30+ or <10 | C | 1.8 | Group of 1 elder and 5 young Ash, dimensions of largest provided; Ash at risk of Ash Dieback Disease | No work at present | Small group, easily replaced |
| G4 | Ash group | 14 | N 2 E 2 S 2 W 2 | 6 | Y | 30+ or <10 | C | 1.7 | Group of 7 young Ash, dimensions of largest provided; 4 of 7 in decline with Ash Dieback Disease, remaining 3 at risk | No work at present | Small group, easily replaced |

Central Woodland

| NO | SPECIES | D | SP | HT | AGE | LIFE | CAT | RPA | DESCRIPTION | RECOMMENDATIONS | COMMENTS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| cw1 | Willow | 70 | N 0 E 8 S 8 W10 | 22 | LM/S | <10 | C/U | 8.4 | Large heavy mature tree; past bough breakage; split bough; significant dead wood in crown; infected with Honey fungus with significant areas of necrotic and missing bark | Prune to make safe Retain for habitat and manage decline | In obvious decline; unlikely to recover; within failing distance of path; streamside |
| cw2 | Willow | 45+ 40+ 40+ 35 | N 8 E 7 S 6 W 7 | 20 | LM | <30 | B | 9.6 | Large heavy mature tree; forked at base; weak branching angles; large dead bough | Dead wood (for safety) | Close to path |
| cw3 | Oak | 20+ 18 | N 1 E 7 S 2 W 1 | 7 | M | 30+ | C | 3.2 | Small maturing tree; suppressed; forked at base; past basal damage | No work at present | Understorey tree; easily replaced |
| cw4 | Oak | 40 | N 8 E 5 S 0 W 5 | 15 | M | 30+ | B | 4.8 | Young maturing tree; heavy dead wood in crown | Reduce dead wood in length (to reduce potential to break and encourage decay on tree) | Close to path |
| cw5 | Willow | 55+ 45 | N 6 E 8 S 0 W 3 | 20 | LM | <30 | C/U | 8.5 | Large heavy mature tree; forked near base; southern stem split and liable to collapse into stream and/ or on to path; dead wood in crown | Prune to make safe Retain for habitat and manage decline | Close to path and stream |
| cw6 | Sycamore | 42 | N 8 E 8 S 6 W 6 | 18 | M | 30+ | B/C | 5.1 | Maturing tree; rubbing damage at height from willow cw5; past branch breakage | Inspect rubbing damage to assess risk of breakage towards path | Close to path |
| cw7 | Hawthorn | 15+ 15+ 12 | N 2 E 1 S 1 W 1 | 7 | M | 30+ | C | 2.9 | Small mature tree; ivy clad; largest of group of 5 Elder, 1 Hawthorn and 1 Hazel | No work at present | Close to path; easily replaced |
| cw8 | Hawthorn | 30+ 25+ 25+ 18 | N 5 E 4 S 3 W 4 | 9 | LM | 30+ | B/C | 5.9 | Mature tree; forked at base; untagged Hazel grows at base | No work at present | Close to path |
| cw9 | Willow | 90+ 30 | N 10 E 10 S 8 W 0 | 20 | LM | <30 | B/C | 11.4 | Large heavy mature tree; collapsed in past with basal sweeps to east; past bough breakage; recent bough breakage; dead wood in crown | Prune to make safe Retain for habitat and manage decline | Prominent tree |
| cw10 | Willow | 45 | N 0 E 0 S 6 W 8 | 10 | LM | <10 | C/U | 5.4 | Large heavy mature tree; longitudinal split at base; collapsed in past onto path; now prone with upright growth, latter intrinsically weak and liable to future breakage onto path | Cut at base to regenerate as a coppice stool | Collapsed; ongoing issues with path if retained as is |
| cw11 | Hazel | 40 ag | N 3 E 5 S 3 W 1 | 7 | LM | 20+ | B/C | 4.8 | Largest of group of Hazels | No work at present; consider managing as coppice stools in future | Large shrubs; relatively easily replaced |
| cw12 | Hawthorn | 16 | N 1 E 2 S 5 W 2 | 7 | M | 30 | C | 1.9 | Small mature tree | No work at present | Small; easily replaced |
| cw13 | Willow | 30+ 28 | N 0 E 0 S 4 W12 | 10 | LM | <30 | C | 4.9 | Mature tree; forked at base; collapsed in past towards path; closest of group of similar sized willows | No work at present |  |
| cw14 | Hawthorn | 25 ag | N 1 E 0 S 3 W 3 | 8 | M | 30 | C | 3.0 | Small mature tree; early forking | No work at present | Small, easily replaced |
| cw15 | Hawthorn | 20 ag | N 1 E 1 S 1 W 2 | 7 | M | 30 | C | 2.4 | Small mature tree; early forking | No work at present | Small, easily replaced |
| cw16 | Elder | 15 | N 2 E 1 S 1 W 1 | 5` | LM | <30 | C | 1.8 | Largest of group of elders | No work at present | Large shrub, easily replaced |
| cw17 | Hawthorn | 50 ag | N 3 E 1 S 3 W 2 | 8 | LM | 30+ | B | 6.0 | Mature tree; forked at base | No work at present | Near path |
| cw18 | Hawthorn | 20 ag | N 2 E 2 S 3 W 3 | 9 | M | 30 | B/C | 2.4 | Small mature tree | No work at present | Near path, easily replaced |
| cw19 | Hawthorn | 20 ag | N 2 E 2 S 1 W 2 | 7 | M | 30 | C | 2.4 | Small mature tree; early forking; largest of group of 4 Hawthorn and 1 young Wild cherry | No work at present | Near path; easily replaced |
| cw20 | Hawthorn | 18 | N 2 E 2 S 1 W 1 | 7 | M | 30 | C | 2.1 | Small mature tree | No work at present | Near path; easily replaced |
| cw21 | Hawthorn | 25 ag | N 2 E 2 S 2 W 3 | 7 | M | 30 | C | 3.0 | Small mature tree; largest of group of 6 Hawthorn including along boundary | No work at present | Near path; easily replaced |
| cw22 | Sycamore | 30 | N 3 E 4 S 4 W 2 | 14 | M | 30+ | B/C | 3.6 | Young maturing tree; early forking | No work at present | Promising young tree |
| cw23 | Sycamore | 22+ 18 | N 1 E 3 S 4 W 0 | 14 | M | 30+ | C | 3.4 | Young maturing tree; forked at base; crowded and somewhat suppressed | No work at present | Remove in future to favour Sycamore cw22 |
| cw24 | Sycamore | 60+ 32+ 22+ 16+ 10 | N 8 E 9 S 9 W 6 | 20 | LM | 30+ | B | 8.9 | Large heavy mature treel; forked at base then early forking; ivy prevents full inspection | No work at present | Prominent tree with potential to become an interesting specimen in the future |
| cw25 | Oak | 40 | N 2 E 4 S 3 W 6 | 16 | M | 30+ | B/C | 4.8 | Young maturing tree; sinewy trunk | No work at present | Fairly crowded, near path |
| cw26 | Oak | 38 | N 2 E 2 S 3 W 4 | 18 | M | 30+ | B | 4.6 | Young maturing tree | No work at present | Fairly crowded, near path |
| cw27 | Sycamore | 22+ 20+ 15+ 12+ 11 | N 2 E 5 S 5 W 3 | 16 | M | 30+ | B | 4.4 | Young maturing tree; stool-grown | No work at present | Fairly crowded, near path |
| cw28 | Oak | 25+ 22 | N 2 E 4 S 3 W 4 | 16 | M | 30+ | B | 4.0 | Young maturing tree; forked near base; adjacent untagged smaller oak (18 cm Dbh) at base sharing same root plate | No work at present | Fairly crowded, near path |
| cw29 | Oak | 45 | N 6 E 9 S 10 W 4 | 20 | M | 30+ | B | 5.4 | Mature tree; ingrown secondary stem graft at base; dead wood in crown | No work at present | Fairly crowded, near path |
| cw30 | Oak | 40 | N 4 E 8 S 1 W 1 | 18 | M | 30+ | B | 4.8 | Mature tree; early forking | No work at present | Fairly crowded, near path |
| u/t | Mixed group |  |  |  |  |  |  | Up to 1.44 | Understorey group including 1 small Oak (12 cm Dbh), 3 Hazel stools, Elm and other saplings <5 cm Dbh | No work at present | Near path; easily replaced |
| cw31 | Oak | 25 | N 4 E 3 S 1 W 1 | 16 | M | 30+ | C | 3.0 | Mature tree; crowded and suppressed | No work at present | Near path; limited long term potential |
| cw32 | Sycamore | 40 | N 3 E 3 S 3 W 3 | 18 | M | 30+ | B | 4.8 | Young maturing tree; small oak (20cm Dbh) on west side sharing same root plate | No work at present | Near path; on steep slope |
| u/t | Dead |  |  |  |  |  | U |  | Dead stem east of cw32 closer to path | Reduce to a tall stump 3 m height and retain for habitat (fairly urgent for path safety) | Dead; on steep slope; near path |
| u/t | Oak |  |  |  |  |  | U |  | North-east of cw32 with significant past basal breakage and liable to collapse into stream | Fell (to protect stream flows) | On very steep slope; near stream |
| cw33 | Oak | 25 | N 2 E 2 S 2 W 2 | 16 | M | 30+ | C | 3.0 | Maturing tree; crowded and somewhat suppressed by Willow cw1 | No work at present | Care required when pruning cw1 to minimise damage to cw33 |