PERCOLATION TEST RESULTS

- THESE TESTS ARE CARRIED OUT WITHIN THE PROPOSED INFILTRATION AREA WHICH SHOULD BE AT LEAST 5M FROM THE INTENDED BUILDING AND BOUNDARY.

- HAVE EXCAVATED DOWN AND INTO A 300MM SQUARED SECTION BELOW THE PROPOSED INVERT LEVEL OF THE EFFLUENT DISTRIBUTION PIPE.

- HAVE FILLED THE SQUARE SECTIONS TO OVER 300MM AND LEFT OVERNIGHT , THIS IS TO SATURATE THE SOIL SURROUNDING THE TEST HOLE TO SIMULATE DAY TO DAY CONDITIONS.

- THE FOLLOWING DAY , REFILLED THE TEST SECTIONS WITH WATER TO A DEPTH OF AT LEAST 300MM AND OBSERVED THE TIME (T) IN SECONDS, FOR THE WATER TO SEEP AWAY FROM 75% TO 25% FULL LEVEL

- HAVING AVOIDED ANY SEVERE WEATHER CONDITIONS, THE TEST WAS CARRIED OUT DURING ACCEPTABLE WEATHER CONDITIONS

- IN EVALUATING YOUR TEST RESULTS PLEASE NOTE THAT WHERE THE VP VALUE DOES NOT FALL BETWEEN 15 SECS/MM THEN INFILTRATION TRENCH OR BED SYSTEMS MAY NOT BE POSSIBLE.

| Test Hole | Depth Below Ground Level | Depth of water | Time taken between 75% & 25% full(seconds) (T) | Percolation value VP=T/150 (VP) | Occupant capacity (P) | Minimum Area A=P X VP X 0.25 |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1200MM | 350MM | 3822 | 25.48 | NA | NA |
| 2 | 1200MM | 350MM | 6239 | 41.59 | NA | NA |
| 3 | 1200MM | 350MM | NA | NA | NA | NA |
| 4 | 1200MM | 350MM | 2205 | 14.7 | NA | NA |
| 5 | 1200MM | 350MM | 2810 | 18.73 | NA | NA |
| 6 | 1200MM | 350MM | 3075 | 20.50 | NA | NA |
| 7 | 1200MM | 350MM | 3522 | 23.48 | NA | NA |
| 8 | 1200MM | 350MM | 2990 | 19.93 | NA | NA |
| 9 | 1200MM | 350MM | 3110 | 20.73 | NA | NA |
| 10 | 1200MM | 350MM | 3705 | 24.7 | NA | NA |

Results are an average across tests carried out

Note . test hole 3 was unable to fill to correct level as water would drain to quickly

Site location : 3 Wath Road Barnsley S74 8HJ

Description of ground strata : Gravel and Sand, Peat

Date test carried out : 27th to 29th September 2022

Name of person carrying out test : George Clark

The water table is more than 1m below the bottom of distribution pipes and I further confirm that there are no wells, springs or water abstraction points within 100m of the proposed infiltration area.

Map Of test holes