| Combined RS 1&2 Problem |  | 7.1 |  |  |
| --- | --- | --- | --- | --- |
| Hazard |  | Sculpture and plinth located adjacent to the westbound running lane of the A635 Barnsley Road. |  |  |
| Risk Description |  | Vehicles losing control and colliding with sculpture and/or plinth |  |  |
| RSA Recommendation |  | It is recommended that a RRRAP assessment be undertaken on the scheme to determine whether or not any additional vehicle restraint measures are required in the vicinity to prevent the potential for vehicles to collide with the sculpture in the even that they were to leave the carriageway. |  |  |
| Likelihood (L): 2 | Severity (S): 3 |  |  | Risk Value (R): 6 (Low) |
| Discussion |  |  |  |  |
| The RSA Team believe that should a vehicle lose control at this location they are likely to leave the carriageway and collide with the proposed sculpture. However, given the location of the structure in relation to the carriageway edge the likelihood of this occurring has been scored as 2 (unlikely). Existing mitigations include a reduction in speed limit to 50mph, running lane narrowing through hatching to reduce vehicle speeds further, highway signing to warn of the roundabout and a relatively new carriageway surface with an increase PSV. Vehicles will be reducing their speed on the approach to the roundabout, and coupled with the location of the sculpture from carriageway edge, the severity of harm has been scored as 3 (serious) as it is unlikely any death will occur as a result of possible collisions. |  |  |  |  |
| Conclusion |  |  | This results in a Risk Value Score of 6 (Low). It can be assumed that the control measures, as designed (sculpture placed 13m from carriageway edge) reduce the risk to an acceptable level. |  |
| Risk Classification |  |  | Low (1–9) Ensure assumed control measures are maintained and reviewed as necessary |  |