| RSA Ref: | RSA Problem | Recommendation | Design Org. response | Overseeing Org. response | Agreed RSA Action |
| --- | --- | --- | --- | --- | --- |
| Problem 7.1 | The drawings submitted for audit do not include details of any measures to prevent vehicles from colliding with the sculpture and / or its plinth should they leave the carriageway. In the event that the driver of a vehicle approaching the roundabout or attempting to negotiate the roundabout ICD were to lose control, it could lead to the vehicle concerned leaving the carriageway at speed whereupon the vehicle could collide with the sculpture resulting in injury to vehicle occupants. | It is recommended that a RRRAP assessment be undertaken on the scheme to determine whether or not any additional vehicle restraint measures are required in the vicinity to prevent the potential for vehicles to collide with the sculpture in the even that they were to leave the carriageway. | Disagree. Sculpture to be installed 13m back from carriageway edge at a location where vehicles will be reducing their speeds on the approach to the roundabout. A Risk Assessment has been undertaken as per the requirements of paragraphs 4.16 and 4.17 of GG119 and is included as Appendix B to this RSA Response Report. | Agree with Design Organisation Response following production of Risk Assessment included as Appendix B to this report. | No further action. |