PLANNING CONSULTATION RESPONSE

| Application No | 2025/0513 |
| --- | --- |
| Proposal | Erection of detached garage |
| Address | 16 Maple Close, Kendray, Barnsley, S70 3NN |
| Date of Consultation Reply | 2nd December 2025 |
| Consultee | Highways DC |

| Consultation Assessment and Justification |  |  |
| --- | --- | --- |
| When originally constructed, no.16 Maple Close had the provision of two off-street parking spaces immediately to the side of the property located within a communal parking area that contained eight spaces in total for no.’s 12 to 18 Maple Close. Only one of the two original spaces for no.16 now remains, the rear of the spaces having since been walled-off and incorporated into the rear garden area. The neighbouring property no.14 has had the entire front garden area converted to a frontage parking facility, the residents therefore presumably seldom used their allocated spaces within the communal parking area adjacent to those of no.16. As such, it is stated within the submitted information that the applicant (owner of no.16) has purchased the spaces; the area of land is therefore included within the red line boundary of this application. The ’site plan’ drawing shows that within the parking area, it is proposed to erect a detached garage with dimensions of 5m x 6m which is of sufficient size to be classified as one parking space. No.16 requires the provision of two off-street parking spaces, however, the remaining tarmac surfaced area to the front of the proposed garage is insufficient to accommodate a second parking space of the required 2.5m x 5m dimensions. The applicant should note that, in order to comply with the Barnsley Supplementary Planning Document (SPD) Parking (2019), where a space is to be provided within a garage, in order for it to be classed as a parking space it must have minimum internal dimensions of 3m x 6m for a single garage or 6m x 6m for a double garage, the proposed garage is therefore too small to be counted as two parking spaces. It should also be noted that the minimum forecourt depth (the length of driveway for a parking space in front of a garage) is 6m for garages with an up and over door and 5.5m for garages with a roller shutter door. It should therefore be demonstrated that two off-street parking spaces of sufficient dimensions, are to be provided within the site curtilage. Should the applicant wish to address this issue I shall be happy to provide further comments as necessary. |  |  |
|  | Defer for amends/further information |  |
| Consultation Suggested Conditions: |  |  |
| Consultation Informative(s): |  |  |
| Planning Obligations required: |  |  |