PLANNING CONSULTATION RESPONSE

| Application No | 2026/0640 |
| --- | --- |
| Proposal | Residential development of 270 new dwellings, together with access, public open space, landscaping, drainage infrastructure and associated works |
| Address | Land south of Cemetery Road, Jump, Barnsley, S73 0QN |
| Date of Consultation Reply | 15/09/2026 |
| Consultee | Pollution Control – Contaminated Land |

| Consultation Assessment and Justification |  |  |
| --- | --- | --- |
| The associated documentation has been reviewed and the location and risks have been assessed and my comments are as follows: The Preliminary Geoenvironmental Investigation indicates that a thorough ground investigation is required which aims to characterise the properties of natural soils, determine the depth and extents of any shallow underground workings, and the depth, extents and nature of the former opencast including the line and geometries of buried highwalls. The works should comprise: c. 50 trial pits to determine shallow ground conditions and enable detailed logging of natural and made ground with 6 soakaway tests. c. 10 trial trenches to determine the location of the highwall. c. 6 cable percussive boreholes to retrieve geotechnical data and samples from depth & install gas & groundwater wells. c. 16 rotary openhole probeholes to confirm shallow underground workings do not pose a significant threat to surface stability, and to determine the depth profile of the former opencast, and to allow installation of shallow gas monitoring wells. c. 10 groups of stitches to determine the line and profile of the highwalls. c. 4 rotary cored boreholes to retrieve samples of rock from the base of the opencast for detailed logging. Monitoring and completion of a Hazardous Gas Risk Assessment is recommended in order to determine appropriate gas protection measures for the proposed dwellings. |  |  |
| NO OBJECTION* |  |  |
| *Delete as applicable |  |  |
| The ground investigations and monitoring detailed in the report “Preliminary Geoenvironmental Investigation – Land south of Cemetery Road, Jump, Barnsley – For Barratt and David Wilson Homes”, produced by Lithos, May 2026, ref: 5693/1A, shall be undertaken and a verification report provided to the Local Planning Authority on completion. Reason: To reduce or remove adverse impacts on health and quality of life, especially for people living and/or working nearby, in accordance with Local Plan Policy POLL1 Following ground investigation works a report shall be provided to the Local Planning Authority which includes an appraisal of remedial options, proposal of the preferred option(s) and a remediation statement summarising the works to be undertaken (if required) and a timetable of works. Reason: To reduce or remove adverse impacts on health and quality of life, especially for people living and/or working nearby, in accordance with Local Plan Policy POLL1 On completion of remediation works and prior to occupation of the development, a verification report shall be provided to the Local Planning Authority which confirms the site has been adequately remediated in accordance with the remediation statement. Reason: To reduce or remove adverse impacts on health and quality of life, especially for people living and/or working nearby, in accordance with Local Plan Policy POLL1 |  |  |
| Consultation Informative(s): It is recommended that measures are taken to prevent a nuisance/or affect the quality of life of local residents. Please note that the Council’s Pollution Control Team have a legal duty to investigate any complaints about noise, smoke, odour, light or dust. No waste should be burnt. If a Statutory Nuisance is found to exist, they must serve an Abatement Notice under the Environmental Protection Act 1990. Failure to comply with the requirements of an Abatement Notice may result in a fine of up to £20,000 upon conviction in Magistrates' Court. It is therefore recommended that you give serious consideration to the steps that may be required to prevent a noise, light, odour, dust or smoke nuisance from being created. |  |  |
| Planning Obligations required: n/a |  |  |