PLANNING CONSULTATION RESPONSE

| Application No | 2025/0831 |
| --- | --- |
| Proposal | Change of use and conversion of vacant village club (Use Class F1e) to three residential units (Use Class C3). Including demolition of existing single storey extension. |
| Address | Brierley Village Club, Church Street, Brierley, Barnsley, S72 9JG |
| Date of Consultation Reply | 30th October 2025 |
| Consultee | Highways DC |

| Consultation Assessment and Justification |  |  |
| --- | --- | --- |
| The proposals involve the creation of 3no. five-bedroom dwellings; in order to comply with the Barnsley Supplementary Planning Document (SPD) Parking (2019) Table 1, two off-street parking spaces should be provided per dwelling which equates to a total of six spaces being required. A total of six parking spaces are shown on the proposed site layout plan, however, all of the spaces are undersized, particularly spaces 1 and 2 which measure 2.3m x 4.5m. All spaces should achieve minimum dimensions of 2.5m x 5m. Spaces 1 and 2 are difficult to access and there is no means for vehicles using these spaces to turn within the site due to being positioned immediately adjacent to the site access which has a width of only 3.4m; vehicles require a clear 6m of manoeuvring room to adequately access/egress a parking space when turning within the site. Given that Church Street is a classified road (C3) there is a requirement for vehicles to enter and exit the site in a forward gear in the interests of highway safety. Classified roads by their very nature are busier roads and it is considered that vehicles reversing to/from the site access would have a detrimental impact on highway safety for oncoming vehicles on Church Street. It is also unclear whether spaces 3 to 6 have sufficient room to turn within the site. The ‘Proposed Block Plan’ shows the rear of the spaces as being within 2m of the red line boundary which would render the spaces inaccessible. I note however that the red line boundary shown on the ‘Proposed Block Plan’ differs from that shown on the site location plan, this issue should therefore be clarified. As the proposals currently stand, they are considered unacceptable from a highways development control perspective as they are viewed as being prejudicial to highway safety contrary to Local Plan Policy T4 New development and Transport Safety. |  |  |
|  |  | OBJECT |
| Consultation Suggested Conditions: |  |  |
| Consultation Informative(s): |  |  |
| Planning Obligations required: |  |  |