- Project details

Table 1 Project details

| Report title: | Stage 1 Road Safety Audit Lowfield Road Railway Bridge Signalisation |
| --- | --- |
| Date: | December 2021 |
| Document reference and revision: | 211214-MM-Lowfield Road RSA 1 |
| Prepared by: | Road Safety Initiatives |
| On behalf of: | Gleeson Regeneration And Homes |

Table 2 Authorisation sheet

| Project: | Proposed Residential Development, Land off Lowfield Road, Bolton upon Dearne – Phase 3 |
| --- | --- |
| Report title: | Stage 1 Road Safety Audit Response Report |
| Prepared by: |  |
| Name: | Martin Crabtree |
| Position: | Associate |
| Signed: |  |
| Organisation: | Bryan G Hall |
| Date: | 15th December 2021 |
| Approved by: |  |
| Name: | James Pointon |
| Position: | Director |
| Signed: |  |
| Organisation: | Bryan G Hall |
| Date: | 15th December 2021 |

2 	Introduction

This is the designers Response report for the Stage 1 Road Safety Audit dated 14th December 2021 for the proposed highway works to signalise the Lowfield Road Railway Bridge as part of the Proposed Residential Development, Land off Lowfield Road, Bolton upon Dearne – Phase 3.

3 	Key personnel

Table 3 Key personnel

| Overseeing Organisation: | Wayne Lake on behalf of Barnsley Metropolitan Borough Council |
| --- | --- |
| RSA team: | Martin Morley and Dean Barker of Road Safety Initiatives LLP |
| Design organisation: | Bryan G Hall Ltd |

4	Road safety audit decision log

Table F.4 Road safety audit decision log

| RSA problem | RSA recommendation | Design organisation response | Overseeing Organisation response | Agreed RSA action |
| --- | --- | --- | --- | --- |
| Problem 2.1.1: Potential for debris associated with an errant vehicle to foul the railway line. | It is recommended that the risk of debris fouling the track is assessed and a mitigation measure is included in the design, if appropriate. ‘Managing the accidental obstruction of the railway, road vehicles’ (DfT, September 2020) is a useful reference. | The scheme that was subject to the audit included Trief style kerbs on the approach to and over the railway bridge. These have been extended on the southern side of the eastbound approach to be in line with the Network Rail scheme for the railway bridge and is shown on drawing numbers 20-237-DE-1300-001 Rev B and 20-237-DE-1300-002 Rev B. Network Rail will be consulted by the Local Planning Authority on the proposed scheme who would be responsible, to carry out an independent risk of debris fouling the track assessment in conjunction with the Local Highway Authority. |  |  |
| Problem 2.1.2: Potential for overtaking and head-on conflicts. | It is recommended that the risk of overtaking is mitigated e.g., by providing ‘prohibit’ double-white-lines along both approaches to the traffic signal stop lines. | The proposed scheme has been updated to show ‘prohibit’ double-white-lines along both approaches to the traffic signal stop lines on drawing numbers 20/237/DE/1300/001 Rev B and 20-237-DE-1300-002 Rev B. |  |  |
| Problem 2.2.1: Potential for a conflict involving a cyclist and an overtaking vehicle. | It is recommended that there is a mitigation measure for the potential ‘squeeze’ problem for cyclists. The running lane could be made narrower so that vehicle drivers were discouraged from trying to pass cyclists or made wider to allow for safe overtaking. Suitable warning signs or markings could also be provided. | The proposed scheme has been updated to show a 3.0m running lane in line with LTN1/20 across the bridge to allow cyclists to take up the primary position and to discourage vehicle drivers from attempting to pass a cyclist on drawing numbers 20/237/DE/1300/001 Rev B and 20-237-DE-1300-002 Rev B. Suitable warning signs or markings will be provided and agreed at the detailed design stage. |  |  |
| Problem 2.2.2: Potential for a conflict involving a pedestrian crossing the access road. . | It is recommended that a safe, accessible, and convenient crossing is provided across the un-named access road. | The proposed scheme has been updated to show amendments to the footway on the south side together with dropped kerbs and tactile paving on drawing numbers 20/237/DE/1300/001 Rev B and 20-237-DE-1300-002 Rev B. |  |  |

5 	Design Organisation and Overseeing Organisation statements

Table 5 Design Organisation statement

| On behalf of the design organisation I certify that: 1) the RSA actions identified in response to the road safety audit problems in this road safety audit have been discussed and agreed with the Overseeing Organisation. |  |
| --- | --- |
| Name: | Martin Crabtree |
| Signed |  |
| Position: | Associate |
| Organisation: | Bryan G Hall Ltd |
| Date: | 15th December 2021 |

Table 6 Overseeing Organisation statement

| On behalf of the Overseeing Organisation I certify that: 1) the RSA actions identified in response to the road safety audit problems in this road safety audit have been discussed and agreed with the design organisation; and 2) the agreed RSA actions will be progressed. |  |
| --- | --- |
| Name: | Wayne Lake |
| Signed: |  |
| Position: |  |
| Organisation: | Barnsley Metropolitan Borough Council |
| Date: |  |