| Site Name | HSBC SYGDC Tankersley |  |
| --- | --- | --- |
| Survey Date: | 11/11/2024 |  |
| Ecologist: | Emma McCartney |  |
| Summary of proposed works: | Installation of a ventilation system to make the building more energy efficient. |  |
| Survey limitations (inc. significance): | N/A |  |
| Return visit required to complete PEA?: | No return visit required |  |
| Brief Summary of on-Site Habitats: | Onsite habitats comprise built-up areas and gardens (introduced shrub), ornamental pond, one tree, modified grassland, developed land, sealed surfaces (The SYGDC building and the paved footpaths around the pond/building) . The wider area consists of industrial and commercial properties, along with deciduous woodland areas. |  |
| Additional information | It is understood that the specific location of the BNG enhancement area can be changed if needed. |  |
| Ecological Constraints | Summary | Further survey required? - |
| Statutory Designated Sites | Presence of one Local Nature Reserve located 400 m northeast of the Site. | No further survey required due to the distance of the statutory designated site from the Site and the small scale nature of the works. |
| Non-Statutory Designated Sites | Record centre data will be reviewed once received. | Record centre data will be reviewed once received. |
| Badger signs/setts | No evidence of badger (e.g., sett, dung pits, badger hairs etc.) were recorded within the Site during the site visit. The deciduous woodland to the west of the Site offers suitability for badger sett creation and foraging, however, is separated from the Site by a large security fence. | Record centre data will be reviewed once received. Best practice measures to be implemented during the works. |
| Amphibians/reptiles | Habitats on Site offer limited suitable foraging and sheltering opportunities for amphibians and reptiles, within the shrub habitats along the embankments. Piles of rubble found under the shrubs on Site may provide suitable refugia/hibernacula for amphibians and/or reptiles. | Record centre data will be reviewed once received. Best practice measures to be implemented during the works. |
| Are there potential GCN waterbodies within 250m? | One large pond is located within 250 m of the Site (approx. 150 m northwest) The ornamental pond on Site was dry at the time of the survey therefore does not offer any opportunities for amphibians unless it is filled back up with water. Following a review of MAGIC, two GCN European Protected Species Applications were returned at 380 m north and 380 m west of the Site pertaining to the destruction of a resting place. The Site is also within the Orange DLL Risk Zone. | Record centre data will be reviewed once received. A DLL enquiry form will be submitted. |
| Potential roosting features for bats | The tree located within the Site boundary is ivy clad, however, this feature has been considered to be of negligible suitability due to the age of the tree and the density of the ivy. The Site offers some suitability for foraging and commuting bats in the form of sections of dense scrub. The Site also offers connectivity to foraging and commuting habitat for bats within the wider landscape. Two European Protected Species Applications were returned with the closest one being approximately 1 km northeast, allowing the damage of a brown long-eared, common pipistrelle and whiskered bat resting place between 2020 and 2025. | Record centre data will be reviewed once received. Due to the negligible suitability of the tree for roosting bats, further surveys are not required. The Site has suitability to support foraging and commuting bats. If required, vegetation clearance should be kept to a minimum. Should works be conducted at night they should proceed under a sensitive lighting scheme. |
| Breeding birds - is veg clearance proposed during breeding bird season (Feb-Aug)? | The shrubs and tree present onsite provide suitable habitat for nesting birds. | No further survey is required unless vegetation clearance is later identified to be required on the shrubs and tree during the peak nesting bird season (March – August inclusive). |
| Invasive non-native species | Rhododendron was identified within all the areas of shrub. | No further survey required. Biosecurity measures to be put in place in order to reduce the spread of invasive species. |
| Adjacent waterbodies and watercourses | No watercourses are present within the Site and the ditches located within 250 m and 500 m of the Site are not hydrologically connected. No evidence of otter holts or field signs of water voles were observed within the Site boundary. The Site does not offer any suitability for otter or water vole. | Record centre data will be reviewed once received. |
| Suitable habitat for additional protected species. | Suitable habitat for hedgehog is present amongst the shrubs. | Record centre data will be reviewed once received. |