PLANNING CONSULTATION RESPONSE

| Application No | 2025/0597 |
| --- | --- |
| Proposal | Conversion of loft to form additional flat with new rear external stairs, rear dormer and rooflights |
| Address | 97 High Street, Wombwell, Barnsley, S73 8HS |
| Date of Consultation Reply | 6th August 2025 |
| Consultee | Highways DC |

| Consultation Assessment and Justification |  |  |
| --- | --- | --- |
| The site is within the local centre of Wombwell and as such it benefits from having a large number of amenities within easy reach along with good links to public transport facilities – there is a bus shelter almost directly opposite the site. The location can therefore be considered to be sustainable which complies with NPPF 9 Promoting Sustainable Transport policies and Barnsley Local Plan (January 2019) Policy T3 New Development and Sustainable Travel. Within the vicinity of the site, there are parking restrictions in the form of double yellow lines along both sides of High Street as well as nearby streets including Snowden Terrace and Cemetery Road. Within close proximity of the site (approx. 110m) there is a small public car park off Cemetery Road which offers free parking. The Barnsley Supplementary Planning Document (SPD) Parking (2019) Table 1 states that 1 car parking space should be provided for a 1-2 bedroom dwelling; the site layout plan does not indicate that any off-street parking is to be provided for the proposed flat. Notwithstanding the lack of parking provision, given the sites accessibility to more sustainable transport options, it is not considered to be detrimental to the proposals for this requirement not to be met. Given the aforementioned sustainable location of the site, walking and cycling should be encouraged as a form of active travel and an alternative to the private motor car. In this regard, the proposals should comply with the Barnsley SPD Parking (2019) Table 2 Cycle Parking Standards which requires that 1 secure and undercover cycle parking space is provided per dwelling; this can be dealt with by way of a planning condition. The proposed site plan identifies a bin storage area within the rear yard. Refuse collection arrangements will presumably be the same as those already in place for the existing first floor flat. In view of the above, the proposals are considered acceptable from a highways development control perspective. Should you be minded to grant permission, please include the condition below. |  |  |
| NO OBJECTION |  |  |
| Consultation Suggested Condition: The development hereby permitted shall not commence until a scheme for the parking of bicycles complying with LTN 1/20 Cycle Infrastructure Design has been submitted to and approved in writing by the LPA. The scheme shall be fully implemented before the development is first occupied (or brought into use) and thereafter retained for this purpose. Reason: In the interests of encouraging the use of sustainable modes of transport in accordance with Local Plan Policy T3. |  |  |
| Consultation Informative(s): None. |  |  |
| Planning Obligations required: None. |  |  |