The site comprises of a row of 32 trees, planted close together in a row, presumably originally planted to screen the Territorial army buildings from the adjacent highway. The trees are approximately 50 years old and

Two trees are considered as class A trees T32 and T31, these trees have had space to develop and grow into large well branched specimens.

Unfortunately, due to the close planting the majority of the trees T2 to T30 have not had space to develop and as a result have very mis- shaped crowns and are considered class B trees. This group of trees are of moderate quality with an expected life expectancy of at least 20 years

Previous pruning to reduce some branches that extend over the car park and removal of dead wood has exacerbated the problem with poorly shaped crowns, and the large amount of growth to the Western/roadway aspect is now becoming an issue. The sheer length and height of the majority of the long thin branches is causing problems with the structural integrity of the trees. The pruning is expected to produce a more well branched shape to the trees and thus retain well shaped the trees in the landscape.

To maintain the trees in a safe condition it is desirable to reduce the height and spread of the crowns of the trees, to prevent biomechanical failure and protect the highway and adjacent footpath, as well as the carparking area of the Territorial army base.

| Tree Ref No | Age Species Latin name | Height (m) | Crown Height (m) | Stem Diam (cm) | Crown Spread Metres N. W; E. S. | Observations | Recommendations | Physiological condition Structural condition | Amenity value | Life expectancy | Retention Category | RPA |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T1 |  |  |  |  |  | Planning permission already in place to remove tree following biomechanical failure and severe damage caused to roof of adjacent building. Large amount of weight from this tree now resting on T2 | Tree to be removed with one replacement tree as per planning permission. | Poor Poor |  |  |  |  |
| T2 |  | 18m | 5m | 39cm | 2. 8; 7. 3. | Tree has 4 main stems with very little lateral branching. I large limb previously reduced over building Damage caused by collapse of limb from T1 | Reduce height of tree and length of limbs to East and West by 3m or to max 100mm diameter to reduce biomechanical stress and prevent further collapse onto building/highway. | Fair Poor |  |  | B |  |
| T3 |  | 16m | 7m | 42cm | 4. 6; 7. 3. | 3 main stems one previous reduction towards building/carpark | Reduce height of tree and limbs to East and West by 2m or to max 100mm diameter to reduce biomechanical stress and prevent collapse onto car park/building/highway. | Fair Poor |  |  | B |  |
| T4 |  | 15m | 8m | 39cm | 6. 4; 10. 3. | 4 main stems at 6 m. large overhang to car park/building | Reduce height of tree and limb to East by 2m or to max 100mm diameter to reduce biomechanical stress and prevent collapse onto highway/building. | Fair Poor |  |  | B |  |
| T5 |  | 15m | 8m | 40cm | 3. 9; 2. 2. | Tree forks at 2m with 6 main stems. 3 stems to carpark previously reduced by half. One dead branch hung up in crown | Reduce height of tree and limb to East by 2m or to max 100mm diameter to reduce biomechanical stress and prevent collapse onto highway. Remove dead hung up branch from crown | Fair Poor |  |  | B |  |
| T6 |  | 9m | 6m | 50cm | 4. 9; 7. 3. | Lot of dieback dead wood in limb to West to roadway.. Overhang to Car park and roadway | Reduce limb to East and west by 3m or to max 100mm diameter to reduce biomechanical stress and prevent collapse onto highway/car park | Poor Poor |  |  | B |  |
| T7 |  | 9m | 8m | 25cm | 3. 4; 3. 4. | 4 main stems, 3 previously reduced. I stem has died back. Very poor-quality tree. | Reduce height of tree by 2m Remove all dead wood | Fair Poor |  |  | C |  |
| T8 |  | 13m | 9m | 50cm | 2. 6; 4. 1. | 7 large limbs. Limbs to car park previously reduced by 50%. Overhang to roadway. | Reduce height of tree by 3m Overhang to roadway reduce length of limb by 2m | Fair Fair |  |  | B |  |
| T9 |  | 12m | 9m | 40cm | 2. 4; 3. 2. | Poor shaped crown due to competition from adjacent trees.3 major limbs, 2 previously reduced by 50% over car park. | Reduce height of tree by 2m | Fair Poor |  |  | B |  |
| T10 |  | 15m | 8m | 45cm | 2. 8; 1. 2. | Central stem large dieback | Reduce height of tree by 3m or to max 100mm diameter. Remove central stem back to live wood. Reduce overhang to roadway by 3m or to max 100mm diameter | Fair Poor |  |  | B |  |
| T11 |  | 16m | 8m | 50cm | 4. 8; 4. 3. | 1 Limb to carpark previously reduced. Dead wood in crown | Reduce height of tree by 3m or to max 100mm diameter. Remove dead wood from crown. Reduce overhang to west by 3m or to max 100mm diameter | Fair Poor |  |  | B |  |
| T12 |  | 16m | 8m |  | 4. 7; 3. 4. | 6 main limbs 2 previously reduced to Car park. Lot of minor die back to crown. Reduce limbs to West/roadway by 3m | Reduce height of tree by 2m or to max 100mm diameter. Reduce limbs to West/roadway by 3m | Poor Poor |  |  | B |  |
| T13 |  | 17m | 8m | 40cm | 3. 9; 5. 3. | Large overhang to West/roadway. Dead wood in most limbs I previously reduce limb has further die back | Reduce limbs to West/roadway by 3m. Reduce height by 3m or to max 100mm diameter | Poor Poor |  |  | B |  |
| T14 |  | 16m | 8m | 35cm | 3. 9; 0. 3. | 2 main stems in tree with die back evident. Large overhang to roadway | Reduce height of tree by 3m or to max 100mm diameter Remove dead wood and reduce overhang to west by 3m or to max 100mm diameter | Poor Poor |  |  | B |  |
| T15 |  | 18m | 9m | 50cm | 5. 10; 4. 4. | 5 main limbs, 2 previously reduced by approximately 50% Tree appears to be in decline. Large overhang to West/roadway | Large overhang to roadway reduce length of limb by 3m or to max 100mm diameter Reduce height by 3m. Remove dead wood. | Poor Poor |  |  | B |  |
| T16 |  | 11m | 9m | 45cm | 2. 6; 3. 2. | 2 main limbs Overhang to roadway. | Reduce limb to West/Roadway by 3m | Fair Fair |  |  | B |  |
| T17 |  | 15m | 9m | 36cm | 2. 10; 5. 3. | 3 main limbs. I previously reduced over car Park by approximately 50% | Reduce limbs to West/roadway by 3m. Reduce height by 3m. | Fair Fair |  |  | B |  |
| T18 |  | 12m | 3m | 35cm | 3. 7; 7. 4. | 3 main limbs at 3 metres. Limb to car park previously reduced. Central limb large amount of die back, tree in decline. | Reduce central limb back to live wood. Reduce limb to East and West by 1.5 m | Poor Fair |  |  | B |  |
| T19 |  | 18m | 10m | 30cm | 4. 9; 3. 5. | Tall thin tree with one large limb large overhang to roadway. | Reduce limbs to West/roadway by 3m Reduce height by 3m or to max 100mm diameter | Fair Fair |  |  | B |  |
| T20 |  | 14m | 8m | 30cm | 2. 9; 6. 5. | Forks to two limbs at 3m. Limb to East/carpark previously reduced and has decay from wound down. Tree in decline | Reduce limbs to West/roadway by 3m or to max 100mm diameter. Remove dead wood. | Poor Poor |  |  | B |  |
| T21 |  | 17m | 7m | 35cm | 3. 9; 3. 5. | 3 stems at 3m. Limb to East/car park dead. Tree in decline. Large overhang to West/roadway | Reduce height of tree by 3m Remove dead wood. Reduce length of limb to West by 3m or to max 100mm diameter | Poor Poor |  |  | B |  |
| T22 |  | 17m | 7m | 45cm | 5. 9; 8. 7. | Central stem dead, lot of minor dead wood. Large overhang to West/roadway | Remove dead wood and reduce limbs to East and West by 3m or to max 100mm diameter Reduce height of tree by 2m | Poor Poor |  |  | B |  |
| T23 |  | 16m | 5m | 55cm | 5. 10; 6. 3. | Large limb to North in decline. Large limb to East/carpark in decline. | Removal of both limbs in decline will leave tree with poorly shaped crown even more imbalanced. Remove dead limbs and reduce limb to west by 2m or to max 100mm diameter | Fair Poor |  |  | B |  |
| T24 |  | 12m | 4m | 55cm | 4. 10; 7. 4. | Central stem previously reduced. Lot of dead wood, tree in decline. | Reduce length of limbs to West and east by 3m or to max 100mm diameter | Poor Poor |  |  | C |  |
| T25 |  | 18m | 5m | 50cm | 3. 9; 7. 4. | Large central limb reduced previously. Large overhang to West and East. | Reduce length of both limbs to West by 3m or to max 100mm diameter and east by 2m or to max 100mm diameter | Fair Fair |  |  | B |  |
| T26 |  | 15m | 6m | 55cm | 3. 10; 4. 0. | Tree in decline with previous limb reduction. | Reduce limb over roadway (West) by 3m or to max 100mm diameter | Poor Poor |  |  | B |  |
| T27 |  | 11m | 7m | 3cm | 0. 9; 0. 3. | Crown of tree misshaped due to proximity to neighbouring trees Previous limb reduction to height. | Reduce overhang to West by 3m or to max 100mm diameter Remove minor deadwood | Fair Fair |  |  | B |  |
| T28 |  | 10m | 5m |  | 2. 2; 2. 5. | Central stem previously reduced. Poor form. Overhang to South/PROW | Minor reduction length of limb to South by 2m or to max 100mm diameter | Poor Poor |  |  | B |  |
| T29 |  | 15m | 5m | 30cm | 1. 2; 1. 5. | Poorly formed crown. Overhang to South/PROW | Reduce length of overhang to PROW /South by 2m or to max 100mm diameter | Fair Fair |  |  | B |  |
| T30 |  | 13m | 5m | 30cm | 2. 2; 2. 6. | Poor form, mis shaped crown tree overshadowed by T31. Overhang to South/PROW | Reduce length of overhang to PROW /South by 2m or to max 100mm diameter | Fair Fair |  |  | B |  |
| T31 |  | 21m | 5m | 50cm | 8. 6; 6. 7. | Well branched crown. Overhang to South/PROW | Well branched crown. No Works required. | Good Fair |  |  | A |  |
| T32 |  | 28m | 5m | 75cm | 9. 8; 8. 10. | Large specimen tree with well branched crown. Limb at 5m to South has previously been reduced minor decay at wound, now regrown. Remove limb from over adjacent PROW. Lower limb to North has minor decay | Remove lowest limb from over adjacent PROW to Branch Bark ridge. Lower limb to North prune back to Branch bark ridge. | Good Fair |  |  | A |  |