PLANNING CONSULTATION RESPONSE

| Application No | 2025/0980 |
| --- | --- |
| Proposal | Erection of commercial building with mixed use Classes E(a), E(c) and E(g). |
| Address | 1 - 3 Church Street, Staincross, Barnsley, S75 6BU |
| Date of Consultation Reply | 16/12/2025 |
| Consultee | Pollution Control |

| Consultation Assessment and Justification |  |  |
| --- | --- | --- |
| The associated documentation has been reviewed and the location and risks have been assessed and my comments are as follows: - The Phase 1 Preliminary Contamination Risk Assessment indicates that further works are required, pre-commencement. These works are as follows: - Undertake a series of shallow percussion boreholes, including appropriate in-situ geotechnical testing to help aid with future foundation designs. - Undertake a series of rotary open boreholes to confirm the depth and condition of the Stanley Main, Lower Coal. - Undertake appropriate geotechnical laboratory classification testing, dependant on the soils/deposits encountered below the site area, and pH and soluble sulphate testing. - Undertake appropriate confirmatory contamination screening on selected samples of made ground recovered from site to confirm the suitability of these deposits to remain onsite within a proposed end-use of Commercial. The samples of soil collected should also be forwarded to a UKAS and MCERTS accredited laboratory. - Production of a Phase II Ground Investigation Report, to include Ground Contamination and Ground Gas Risk Assessments. Pollution Control will require a report which confirms the outcome of the above and any remedial works required with timescales. A verification report will also be required following the works. - There is information regarding any fixed external plant associated with the proposed development. Can the applicant confirm whether fixed external plant will be installed? - Conditions restricting times of construction works will be required, as below. |  |  |
|  | Defer for amends/further information* |  |
| *Delete as applicable |  |  |
| Consultation Suggested Conditions: During works, construction or demolition related activity shall only take place onsite between the hours of 0800 to 1800 Monday to Friday and 0900 to 1400 on Saturdays and at no time on Sundays or Bank Holidays Reason: To reduce or remove adverse impacts on health and quality of life, especially for people living and/or working nearby, in accordance with Local Plan Policy POLL1 There shall be no burning of any material on the development site during the construction phases. Reason: To reduce or remove adverse impacts on health and quality of life, especially for people living and/or working nearby, in accordance with Local Plan Policy POLL1 |  |  |
| Consultation Informative(s): It is recommended that measures are taken to prevent a nuisance/or affect the quality of life of local residents. Please note that the Council’s Pollution Control Team have a legal duty to investigate any complaints about noise, smoke, odour, light or dust. No waste should be burnt. If a Statutory Nuisance is found to exist, they must serve an Abatement Notice under the Environmental Protection Act 1990. Failure to comply with the requirements of an Abatement Notice may result in a fine of up to £20,000 upon conviction in Magistrates' Court. It is therefore recommended that you give serious consideration to the steps that may be required to prevent a noise, light, odour, dust or smoke nuisance from being created. |  |  |
| Planning Obligations required: n/a |  |  |