# Residential Development off

# Bolton on Dearne, Phase 2 development

## Drainage Statement

# Report date 26-07-13

# PSGBS

# Residential Development off Lowfield Road, Bolton on Dearne Phase 2

## Drainage Statement

Gleeson Developments Limited intends to construct a new residential development on the land comprising 64 dwellings. There is to be a mixture of detached and semi-detached type properties. The development will be served by new estate roads and sewers which will be offered for adoption by Barnsley Metropolitan Borough Council and Yorkshire Water respectively.

Site Description

The site is located immediately to the south of the current Gleeson Homes residential estate off Lowfield Road National Grid Reference 445917,402347 and comprises a single piece of land approximately rectangular in shape with an area of approx. 2.1ha.

The site is bounded by the existing Gleeson estate to the North, an existing railway line to the West, a sewerage treatment works to the south and open fields to the east.

Existing Drainage

There are no public sewers within the development other than the new sewers that have been laid by Gleeson Homes as part of their Phase 1 development. The sewers from this site are the subject of a section 104 agreement and will therefore be adopted by Yorkshire Water. A separate network of both foul and surface water sewers have been installed for this development.

The site was previously part developed as a flat pack furniture factory with the Phase 2 site sat over what was previously the factories car park and open field area.

Ground Conditions

We have undertaken a Stage 1 and Stage 2 intrusive ground investigation to determine the nature of the ground conditions. This investigation has identified that the soils are generally Clay made ground over Natural clays over sandstone rock head. These conditions are confirmed not to be suitable for soakaway installation.

The sites slopes gradually from north to south at a very shallow gradient although parts of the site are rather rutted and hilly due to the earthworks operation that has taken place on phase 1 development.

Development Proposals

The development proposals have been assessed and we are in discussion with Yorkshire Water the Land Drainage Board and the EA over the most acceptable point of connection and discharge rates. It is intended to construct a new foul water drainage system to convey flows to the previously installed section 104 sewers from phase 1.

The surface water flows from the site will be restricted to the agreed discharge rates and attenuated on site. It is intended to utilise the sewers installed for the phase 1 development as the sites surface water outfall. This is further discussed within the FRA by SKM

Conclusion

A residential development comprising 64 dwellings is to be constructed. It will be served by new estate roads and have a separate system of foul and surface water drainage. The drainage system will incorporate a foul drainage system and attenuation on site to cater for restricted discharge of surface water, both of which are to be offered for adoption by the Water Authority.

Appendix A – Phase 1 site drainage proposals