Ref: 2016/0685

Applicant: Berneslai Homes

Description: Demolition of existing terrace housing and erection of 6 no. bungalows.

Site Address: 1 Beever Street, Goldthorpe, Rotherham

Site Description

This site is located in a mixed use area next to the Goldthorpe town centre, most of which is along Doncaster Road, which to the north of the application site

Beever Street, which is opposite the new Goldthorpe Primary School, comprises primarily high density terraced housing. There is also a car mechanics business and an Elim Church, in a modern single storey brick building.  There are a number of similar terraced streets, including Victoria Street and Cooperative Street, that are linked together by Cross Street.

To the south of Cross Street the predominantly residential character of Beever Street changes to industrial, with three large commercial buildings and external storage.

The 19 terraced houses that occupy the site are currently boarded up and represent a large proportion of one side of the road frontage above Cross Street. There is a break in the terrace and an alleyway provides rear access. To the east of the alleyway are allotment gardens bounded by mature trees.

Proposed Development

The applicants have states that the proposal has the backing of local stakeholders and elected members:

A consultation event for local residents active stakeholder groups and local members was held on the 13 Jan 2016 in a local hall in Goldthorpe. This considered options for the type of property and basic design layout. All participants had the opportunity to discuss their views and or complete a short questionnaire.

There was overwhelming support for the design  and format of development subsequently  submitted.

After demolition of 19 terraced houses the proposal is to erect a single storey development of 6 elderly persons bungalows. The buildings would back onto the rear alleyway presenting a circa 5.5m high blank rear wall punctuated only by small high level windows. A 2.4m high metal mesh fence would be provided to screen the rear elevation from the rear alleyway.

In plan form the development would comprise three interconnected C shaped blocks. Mono pitch roofs would slope into three distinct courtyards. These three courtyards are shown separated by two projecting gable elevations that are connected by a low walls with railings enclosing the courtyards from the street. Along the whole frontage is an area of parking for the units and there are 6 additional spaces at either end of the development. Parking to the sides of the development is proposed as public/ visitor parking and was identified as a requirement through the public consultation. The residents parking would be broken up with landscaping and low galvanised steel fences.

Each bungalow would have 2 bedrooms, a kitchen, living room and ancillary facilities. The arrangement of the dwellings is such that the amenity space is located to the front which provides a more open frontage to the street and opposing properties.

The gardens would contain a large proportion of hard surfaces with wide pathways for wheelchair access. The development is to meet with the requirements of ‘Lifetime homes’ and will therefore be suitable for both elderly and disabled people. Level access to all of the dwellings is key to achieving this requirement.

Policy Context

Planning decision should be made in accordance with the development plan unless material considerations indicate otherwise and the NPPF does not change the statutory status of the development plan as the starting point for decision making.  The development plan consists of the Core Strategy, saved Unitary Development Plan policies and The Joint Waste Plan.  The Council has also adopted a series of Supplementary Planning Documents and Supplementary Planning Guidance Notes, which are other material considerations.

Core Strategy

CSP14 Efficient Use of Land

CSP26 Highways issues.

CSP29 Design matters.

CSP40 Pollution protection.

Saved UDP Policies

Housing Policy Area

SPDs/SPGs

Designing New Housing Development

This sets out space standards and in most situations developments would be expected to comply with the standards. However, the SPD states that full compliance with standards is expected in predominantly residential areas whereas they may be slightly relaxed in town centre situations/higher density areas.

The SPD also states that development must respect local context, however, it is also stated that innovative design solutions that depart from the local context may be allowed if they are considered to have a positive impact on the character of the area.

There is a specific section on Infill development, which states that

As well as the general criteria, including the external spacing standards, infill development should

aim to comply with the following:

- Dwellings should be orientated to have a frontage to the existing public highway. Sides and

backs and garages should be sensitively located so the frontage of the new development

integrates with the existing street scene

- The space between the proposed dwelling and adjacent dwellings should reflect the prevailing character of the street.

- The siting of the dwelling should reflect the building line of the dwellings on the same side of the street.

- The eaves and ridge heights of dwellings should usually be comparable with the heights of

adjacent dwellings.

- Parking provision should be accommodated in a similar manner to how it is accommodated

elsewhere on the street (e.g. if existing dwellings are set forward on their plots with parking

at the side, the proposed dwelling should not be set back with parking at the front).

- Architectural features, fenestration and materials should reflect the positive elements elsewhere on the street.

- Infill development should not be piecemeal so as to prejudice potential comprehensive

development of a larger area of land.

- Landscaped features such as trees, particularly those prominent in the street scene, should

be retained and provided with sufficient space for future growth.

Parking

This sets out requirements for houses in normal occupation. For elderly development there should be one space per unit and communal parking.

Publication Consultation Document

The Council has produced the Publication Consultation Document of the Local Plan. It establishes policies and proposals for the development and use of land up to the year 2033. The document is a material consideration and represents a further stage forward in the progression towards adoption of the Local Plan. As such increasing weight can be given to the policies contained within the document although this is still limited by the need to consider any comments received during the consultation and with the knowledge that the Inspector can require changes to the plan.

SD1 Presumption in favour of sustainable development

GD1 General development

D1 Design

T4 New Development and Highway Improvement

Poll1 Pollution Control and Protection

CC4 Sustainable Drainage Systems (SuDS)

These policies carry little weight for the reason given above. However, the industrial area to the south and surrounding land is designated for housing reflecting a desire to regenerate the area.

NPPF

The National Planning Policy Framework sets out the Government’s planning policies for England and how these are expected to be applied. At the heart is a presumption in favour of sustainable development.

Consultations

Pollution Control-

Highways-

Representations

None received

Assessment

Principle of development

This an infill residential redevelopment in a residential street so the proposals would be is acceptable in principle. The main considerations relate to conformity with Core Strategy policies that seek higher density development in sustainable locations and policies seeking designs that reflect local character and appearance.

This is an area that is subject to significant change, with many of the substandard older terraced housing stock in the area demolished and a new school recently erected nearby. The industrial area to the south is designated for Housing in the Publication Consultation Document of the Local Plan. Whilst this carries little weight it reflects a desire for change. This scheme is of a high quality and has been subject of consultation with local stakeholders. It meets a recognised need for elderly persons accommodation and stands up as a unique design that will add value to the area and provide an uplift in an area where regeneration is sought.

Residential Amenity

As a result of the design concept there would be no private gardens for the future residents and the garden area that is provided would be behind the residents parking. The Council would normally require that family houses have adequate private amenity space to a set standard. However, the bungalows are intended for the elderly so they would meet a special need for level access and this consideration needs to be weighed against normal standards. It is long established that other material planning considerations can allow an exception to be made to normal guidelines.

The C shaped layout gives opposing blocks that would result in living rooms facing each other across a 12m wide courtyard. This could result in internal overlooking at close quarters. However, this is a single storey development so the privacy of occupiers can be protected by boundary fences or walls.  Furthermore, in the existing traditional high density street pattern on Beever Street there are houses facing each other across a road and pavements giving a distance that is only 11m wide.

If the bungalows were to be turned around then the outlook at the rear would be better as there mature trees and allotments to the east but the design is such that there would then be a long high wall with small windows facing the Beever Street. The outlook for existing residents on Beever Street would be degraded.

Therefore, if the gardens were to be relocated a completely new design concept would be required. The design concept has been devised after public consultation that included alternative layouts so there are sound reasons for choosing this approach. As this is not standard family accommodation imposing normal standards for private space is not considered to be essential.

Visual Amenity

The main issue would the fact that the new development is not in the same style and form as the existing high density traditional terraced housing. The Council’s SPD Designing New Housing Development sets out guidance for most situations but in this case it needs to be applied in a more imaginative way. This is a scheme that has been chosen by local stakeholder groups and local members.

Much of the older terraced housing in the immediate area has been demolished and a new primary school has been built. The area is in need of regeneration and this proposal should be judged in these terms.

The existing 19 terraced houses would be replaced with only 6 elderly persons bungalows. It would not be possible or desirable to attempt to mimic the existing style of houses given that this is a complete departure in terms of scale and use.

The arrangement of the dwellings is such that the amenity space is located to the front which provides a more open frontage to the street and opposing properties.

The applicants have commented that amenity space could be positioned at the rear but this would result in a far harder edge being presented to the street. Also due to the mono pitch nature of the dwellings it would result in a significant blank wall being presented to the street.

Therefore, this proposal should be seen as being unique high quality development meeting an identified need that will add value to an area that is subject to change.

Highway Safety

No objections subject to conditions.

Recommendation

Grant subject to conditions