DIGITAL ADVERTISING DIGITAL DISPLAY UNIT

SPECIFICATIONS

| DIGITAL DISPLAYS |  |
| --- | --- |
| Technology | LCD panels |
| Orientation | Portrait |
| Maximum resolution | 1080 x 1920 pixels (Full HD( |
| Active area | 1872mm (H) x 1053mm (W) therefore 1.97sq.m |
| Brightness Control | Automatic Brightness control to adapt screen brightness to ambient light level. Illumination brightness is controlled via a light sensor which monitors ambient light. |
| Maximum brightness in darkness | 280cd/m2 |
| Display operation | Static digital advertising images in sequence, changing no more frequently than every 10 seconds via smooth fade. |
| Networking |  |
| Connectivity | 4G connectivity with roaming access mobile networks. |
| Antenna | External Puck Antenna |
| Physical |  |
| Weight | 280Kg |
| Power |  |
| Power Supply | 220-240 VAC 50Hz single phase mains |
| Environmental |  |
| IP Rating | IP65 |